Advanced oxidation processes: performance, advantages, and scale-up of emerging technologies

Monali Priyadarshinia, Indrasis Dasb,c*, Makarand M. Ghangrekara,c, Lee Blaneyd*

aSchool of Environmental Science and Engineering, Indian Institute of Technology Kharagpur, Kharagpur, West Bengal India, 721302

bEnvironmental Engineering Department, CSIR-Central Leather Research Institute, Adyar, Chennai, Tamil Nadu, India- 600020

cDepartment of Civil Engineering, Indian Institute of Technology Kharagpur, Kharagpur, West Bengal, India, 721302

dDepartment of Chemical, Biochemical, and Environmental Engineering, University of Maryland Baltimore County, Baltimore, MD, USA, 21250

*Corresponding author E-mail: indrasis@clri.res.in (Indrasis Das) and blaney@umbc.edu (Lee Blaney)

Contents

Supplementary S1	3

Review Focus and Novelty Statement	3

Table S1. Previously published reviews on AOPs.	3

Supplementary S2	4

Conventional ozonation process and mechanism	4

Table S2. Transformation of contaminants of emerging concern by catalytic ozonation processes.	5

Supplementary S3	6

Cavitation	6

Table S3. Degradation of contaminants of emerging concern by acoustic cavitation processes.	7

Table S4. A variety of hydrodynamic cavitation processes have been used to degrade organic contaminants.	8

Supplementary S4	9

Table S5. Removal efficiency of emerging contaminants by γ-rays based AOP. Note, C0 is the initial contaminant concentration.	9

Supplementary S5	9

Electrochemical Oxidation based AOP	9

Table S6. Transformation of contaminants of emerging concern by anodic oxidation	12

Supplementary S5	16

Table S7. Degradation of contaminants of emerging concern by various Fenton process	16

Supplementary S8	18

Table S8. Degradation of emerging contaminants by non-thermal plasma assisted AOP	18

# Supplementary S1

# Review Focus and Novelty Statement

# Table S1. Previously published reviews on AOPs.

| No. | Technology | Focus of the work | Reference |
| --- | --- | --- | --- |
| 1 | Coupling of perovskite catalysts and AOPs | Reaction mechanisms of perovskite in AOPs and application | (Lin et al., 2022) |
| 2 | Sonochemical-, persulphate-, photochemical-, and Fenton-based AOPs | Identification of nitrogen-containing byproducts in AOPs, kinetics of nitro-product formation | (Rayaroth et al., 2022) |
| 3 | Plasma activation, catalyst structures, AOP-bioremediation, and membrane-based AOPs | Artificial neural networks (ANNs) for modelling of AOPs, sustainability, challenges | (Giwa et al., 2021) |
| 4 | Ozone-based, Fenton, UV photolysis, H2O2/UV, ozone/UV, photocatalysis, photo-Fenton, and TiO2 photocatalysis | Mechanisms, applications, and recent advancement | (Khare et al., 2021) |
| 5 | Electrochemical oxidation, ultrasonication, and ionizing radiation | Factors and limitations | (Krishnan et al., 2021) |
| 6 | Photolysis, photocatalysis, ozonation, Fenton, anodic oxidation, sonolysis, and wet air oxidation | Mechanisms, factors, degradation products, and effluent toxicity | (Tufail et al., 2021) |
| 7 | Chemical, photochemical, and photocatalytic AOPs | Mechanisms, factors, challenges, formation of toxic disinfection byproducts | (Khan et al., 2020) |
| 8 | H2O2/UV, Fenton, Fenton-like, heterogeneous Fenton, and photo-Fenton, TiO2/UV, TiO2/H2O2/UV, and sonochemical and electro-oxidative AOPs | Mechanisms, challenges, and prospects | (Cuerda-Correa et al., 2020) |
| 9 | Ozonation, UV irradiation, photocatalysis, Fenton reaction, and integrated processes | Process engineering, and environmental, economic, and social parameters | (Fast et al., 2017) |
| 10 | Ozonation, Fenton processes, sonochemical, and TiO2 heterogeneous photocatalysis | Lab-scale applications and mechanisms | (Salimi et al., 2017) |
| 11 | O3, UV, Fenton, and sulphate-based AOPs | Fundamental mechanisms, application to landfill leachate treatment | (Deng and Zhao, 2015) |
| 12 | Chemical, photochemical, sonochemical, and electrochemical reactions | Principles, performance, advantages, drawbacks, and applications | (Oturan and Aaron, 2014) |
| 13 | Radiation, photolysis, photocatalysis, sonolysis, electrochemical, Fenton, and ozone-based processes | Description and performance analysis of AOPs and their application | (Wang And Xu 2012) |
| 14 | O3/UV, H2O2/UV, O3/H2O2, Fe2+/H2O2/UV, US/H2O2, anodic oxidation, electro-Fenton, H2O2/catalyst, catalytic ozonation, photocatalytic, and ozonation | Mechanisms and lab-scale applications | (Poyatos et al., 2010) |

# Supplementary S2

# Conventional ozonation process and mechanism

Strong oxidation potential (2.07 V) of O3 is highly efficient for water disinfection and bacteria destabilization (Hoigne and Bader, 1975). In general, ozonation follows two pathways: direct and indirect ozonation. In direct ozonation, O3 molecules directly react with the organic pollutants via oxidation-reduction reactions (Eq. 1), whereas, in indirect ozonation, several free species, such as OH˙, are generated via series of reactions (Eq. 2-7). At low pH, direct ozonation dominates; whereas, at alkaline conditions, indirect ozonation prevails because of the high decomposition rate of O3 molecules at high pH range (Figueredo et al., 2019).

Under acidic conditions;

3O3 + OH− + H+ → 2•OH + 4O2						(Eq. 1)

Under alkaline conditions;

O3 + OH− → O2 + HO2−							(Eq. 2)

O3 + HO2− → HO2• + O3•−							(Eq. 3)

HO2• → H+ + O2•− 								(Eq. 4)

O2•− + O3 → O2 + O3−								(Eq. 5)

O3 + H+ → HO3								(Eq. 6)

HO3 → •OH + O2								(Eq. 7)

# Table S2. Transformation of contaminants of emerging concern by catalytic ozonation processes.

| Process type | Type of catalyst | Catalyst dose (mg L‒1) | Ozone dose (mg L‒1) | Contaminant | C0 (mg L‒1) | Time (min) | Removal efficiency (%) | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Homogenous | Ce/red mud | 50 | 0.5 | Bezafibrate | 1 | 30 | 96.5c | (Xu et al., 2016) |
|  | Fe2+ | 100 | 380 ± 20 | Beta-Blockers | 0.2 | 120 | 77.9 a | (Wilde et al., 2014) |
|  | Mn2+ | 4.1 | 72×10-2 | 17β-estradiol | 2.7 | 5 | 90 c | (Jiang et al., 2013) |
| Heterogeneous | Fe-MCM-48 | 150 | 100 | Diclofenac | 15 | 60 | 50 b | (Li et al., 2018) |
|  | MnO2 | 40 | 1.7 | Irgarol | 2 | 10 | ~100 c | (Guo et al., 2020) |
|  | Co3O4/C | 50 | 15 | Norfloxacin | 20 | 10 | ~100 c | (Chen et al., 2021) |
|  | CoFe2O4 | 200 | 5.1 | Clofibric acid | 20 | 30 | 98.7 c | (Cai et al., 2021) |
|  | Nano-MgO | 200 | 2 | Quinoline | 20 | 15 | 90.7 c | (Zhu et al., 2017) |
|  | Alumina | 1000 | 4.5 | Bisphenol A | 10 | 60 | 90 c | (Keykavoos et al., 2013) |

a: as chemical oxygen demand

b: as total organic carbon

c: as contaminant removal

# Supplementary S3

# Cavitation

Cavitation is a phenomenon of rapid formation, growth and collapse of microbubbles or cavities in the aqueous medium. However, the collapse of cavities releases large magnitudes of energy within a very small location (Wu et al., 2015). The energy released during rapid collapse of microbubbles in the cavitation systems result in the formation of extremely reactive radicals, which can enhance the complete mineralization of organic pollutants present in the wastewater (Suslick, 1990). The formation of microbubbles or cavities depends upon the temperature and static as well as dynamic pressure of the water (Senthil Kumar et al., 2000).

# Table S3. Degradation of contaminants of emerging concern by acoustic cavitation processes.

| System | Contaminant | C0 (mg L‒1) | Experimental conditions | Time (min) | Removal efficiency (%) | References |
| --- | --- | --- | --- | --- | --- | --- |
| US | Trimethoprim | 0.5 | US frequency = 20 kHz | 90 | 90 | (Arvaniti et al., 2020) |
| US | Benzenesulfonic acid | 15.8 | US frequency = 350 kHz | 275 | 50 | (Thomas et al., 2020) |
| US/H2O2 | Hexabromocyclo-dodecane | 12.8 | US frequency = 20 kHz H2O2 = 20 mg L‒1 | 40 | ~100 | (Panda et al., 2020). |
| US/UV/N/Ti3+ co-doping biphasic TiO2/Bi2WO6 | Methylene blue | 5 | US frequency = 35 kHz UV fluence = 300 mJ cm‒2 Catalyst = 500 mg L‒1 | 50 | 98 | (Sun et al., 2020) |
| US/PS/ nanoscale zero-valent iron | Chloramphenicol | 5 | US frequency = 40 kHz PS = 1 mM Catalyst = 0.5 g L‒1 | 90 | 98.1 | (Zhang et al., 2020) |
| US/UV/TiO2 | Diclofenac | 25 | US frequency = 20 kHz Catalyst = 0.1 g L‒1 UV fluence = 16.8 mJ cm-2 | 80 | ~100 | (Meroni et al., 2020) |
| US/photo-Fenton | Ampicillin | 10.5 | US frequency = 375 kHz Fe = n.a. a | 60 | 80 | (Montoya-Rodríguez et al., 2020) |
| US/WS2@CeO2 | Tylosin | 10 | US frequency = 40 kHz Catalyst = 1 g L‒1 | 40 | 76.5 | (Yousef Tizhoosh et al., 2020) |
| US/UV/MgO/carbon nanotubes | Sulfadiazine | 45 | US frequency = 24 kHz Catalyst = 0.9 g L‒1 | 80 | ~100 | (Hayati et al., 2020) |

a: n.a., not available

# Table S4. A variety of hydrodynamic cavitation processes have been used to degrade organic contaminants.

| System | Contaminant | C0 (mg L‒1) | Experimental conditions | Time (min) | Removal efficiency (%) | References |
| --- | --- | --- | --- | --- | --- | --- |
| HC/Fe3+-doped TiO2 | Rhodamine B | 20 | Pressure = 3 bar Catalyst = 0.5 mg L‒1 | 150 | 91.11 | (Li et al., 2020) |
| HC/UV/O3/H2O2 | Humic acid | 3 | Pressure = 4 bar UV fluence = 300 mJ cm‒2 O3 = 2 mg L‒1 H2O2 = 4 mg L‒1 | 60 | 71.2 | (Čehovin et al., 2017) |
| HC/O3 | Methyl orange | 20 | Pressure = 0.4 bar O3 = 46.32 mg L‒1 | 10 | ~100 | (Wang et al., 2020) |
| HC/PS/O3 | Oxalic acid | 100 | Pressure = 5 bar PS = 2.45 mM O3 = 81 ± 2 g m‒3 | 100 | 72 | (Choi et al., 2019) |
| HC/H2O2 | Dicofol | 50 | Pressure = 7 bar H2O2 = 50 mg L‒1 | 60 | 85 | (Panda and Manickam, 2019) |
| HC/PS/Fe(II) | Pentachlorophenol | 10 | Pressure = 5.77 bar PS = 0.34 mM Fe = n.a. a | 60 | ~100 | (Choi et al., 2020) |
| HC/O3 | Naproxen | 10 | Pressure = 4 bar O3 = 2 g h‒1 | 40 | ~100 | (Thanekar et al., 2020) |
| HC/O3/H2O2 | 2,4,6-trichlorophenol | 20 | Pressure = 4 bar O3 = 400 mg h‒1 H2O2 = n.a. a | 120 | ~100 | (Barik and Gogate, 2018) |
| HC/FeSO4/H2O2 | 2,4-dinitrophenol | 20 | Pressure = 4 bar H2O2 = 0.1 g L‒1 FeSO4 = 0.6 g L‒1 | 60 | ~100 | (Bagal and Gogate, 2013) |

a: n.a., not available

# Supplementary S4

# Table S5. Removal efficiency of emerging contaminants by γ-rays based AOP. Note, C0 is the initial contaminant concentration.

| Process | γ-radiation absorbed dose (kGy) | Contaminant | Co (mg L‒1) | Efficiency (%) | References |
| --- | --- | --- | --- | --- | --- |
| γ- rays /H2O2 | 12 | Reactive Blue-19 | 50 | 60 | (Arshad et al., 2020) |
| γ-rays / Fenton | 1 | Sulfadiazine | 25 | ~100 | (Rivas-Ortiz et al., 2017) |
| γ- rays | 8 | Diclofop methyl | 150 | 97a | (Zaouak et al., 2020) |
| γ- rays /H2O2 | 15 | Nonylphenol ethoxylates | 150 | 90 | (Iqbal and Bhatti, 2015) |
| γ- rays | 5 | Indigo carmine | 150 | 96b | (Zaouak et al., 2018) |
| γ- rays /H2O2 | 3.0 | Ofloxacin | 181 | 99.6 | (Changotra et al., 2018) |
| γ- rays /H2O2 | 25 | 2-Chlorophenol | 100 | 90 | (Basfar et al., 2017) |
| γ- rays /H2O2 | 9 | p-Nitrophenol | 50 | ~100 | (Yu et al., 2010) |

a: as total organic carbon

b: as chemical oxygen demand

# Supplementary S5

# Electrochemical Oxidation based AOP

S.5.1. Direct oxidation

According to this oxidation process the water molecules are oxidized to form the •OH at anode surface (anodic oxidation); whereas, H2 gas is evolved at cathode surface (Dewil et al., 2017). Additionally, electrode materials plays a significant role in enhancing the efficacy of oxidation, degradation pathway and reaction mechanisms of the process (Ahmad et al., 2021a; Salmerón et al., 2020). The electrode materials can be classified into two categories, such as active anode including Pt, IrO2 and RuO2, and nonactive anode including PbO2, SnO2, and boron-doped diamond (BDD) (Zhu et al., 2008).

The surface of metal oxide based active anodes are capable in electrochemical conversion of non-biodegradable organics into more biodegradable form such as short-chain carboxylic acids rather than complete mineralization into CO2; whereas, the surface of nonactive anodes results in complete mineralization of organics into CO2 (Garcia-Segura et al., 2018). This happens in active anodes due to higher oxidation states are available for metal or metal oxide anode above the standard potential for oxygen evolution (E0 = 1.23 V/SHE). Therefore, the absorbed •OH may form physisorbed active oxygen species, such as superoxide (O2•−, MO) (Scialdone et al., 2011). In contrast, in nonactive anodes, the anode acts as a sink for the removal of electrons (Oturan and Brillas, 2007). Hence, a weak interaction exists between the anode surface and •OH and it results in complete mineralization of organics into CO2.

The initial pathway in both types of anodes involves the oxidation of water molecules leading to produce physisorbed active oxygen and metal absorbed •OH on the surface of anodes (denoted as M) (Eq. 8). The surface of active anode absorbed •OH and react vigorously to form MO (Eq. 9) and MO further participate in the oxidation of organic pollutants (R), resulting in the formation of redox couple MO/M as an intermediate reaction (Eq. 10). However, at the same time oxygen evolved from the chemical decomposition of MO as a competition with the side reactions (Eq. 11) (Brillas and Martínez-Huitle, 2015). In the case of nonactive anode, the surface interacts so-weakly with the OH˙ radicals and that allows the direct reaction of M(•OH) with the organic pollutants to gives complete mineralization products such as CO2 (Eq. 12). However, at the same time, this mineralization reaction competes for side reaction of M (•OH) to form O2 (Eq. 13) and H2O2 is generated as an intermediate of the reaction (Eq. 14) (Ahmad et al., 2021b).

M + H2O → M (•OH) + H+ + e−						(Eq. 8)

M (•OH) → MO + H+ + e−							(Eq. 9)

MO + R → M + RO								(Eq. 10)

MO → M + 12 O2 								(Eq. 11)

M (•OH) + R → M + mCO2 + nH2O + H+ + e−				(Eq. 12)

M (•OH) → M + 12 O2 + H+ + e−						(Eq. 13)

2M (•OH) → 2M + H2O2							(Eq. 14)

Direct anodic oxidation is a clean and efficient way to electrochemically generate •OH. The generation of •OH requires mild reaction conditions, such as shorter reaction time, room temperature and non-involvement of any chemicals (Cañzares et al., 2003). As previously mentioned, anode material plays a significant role in enhancing the efficacy of the process. A low cost Ti/SnO2–Sb/Ce–PbO2 anode was fabricated for the removal of 20 mg L‒1 capecitabine from wastewater and almost 93% of degradation was also observed within 7 min by using current density of 10 mA cm-2 (Xu et al., 2020). A novel indium doped PbO2 electrode was used to treat 500 mg L-1  aspirin based wastewater and 76.45% of aspirin removal was achieved within 2.5 h of electrolysis by adopting a current density of 50 mA cm‒2 (Xia et al., 2020b). Furthermore, the application of a novel 3D-hexagonal Co3O4 anode was explicitly engineered for the removal of 10 mg L‒1 of 4-nitrophenol from wastewater. Almost complete mineralization of 4-nitrophenol was observed within 60 min of electrolysis (Zou et al., 2019). Efficient removal of the recalcitrant compounds from wastewater was also investigated by several researchers by employing direct anodic oxidation method and very high removal efficiency were observed (Table S6).

# Table S6. Transformation of contaminants of emerging concern by anodic oxidation

| Anode | Contaminant | Co (mg L‒1) | Time (min) |  | Current density (mA cm‒2) | Efficiency (%) | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ce-PbO2/ZrO2 | Acridine orange | 30 | 90 | 50 |  | ~100b | (Yao et al., 2019) |
| Fe-doped PbO2 | Acid orange 7 | 100 | 60 | 20 |  | 87.15a | (Xia et al., 2020a) |
| BDD | Norfloxacin | 100 | 120 | 83 |  | 92 b | (Mora-Gomez et al., 2019) |
| BDD | Phenol | 1880 | 180 | 5 |  | 90 b | (Kornienko et al., 2011) |
| IrO2 | Chloranilic acid | 450 | 225 | 50 |  | 90 b | (Martínez-Huitle et al., 2004) |
| Cu doped PbO2 | Amoxicillin | 177.3 | 30 | 46.1 |  | 98.4 b | (Hu et al., 2020) |
| Titanium suboxide mesh | Sulfadiazine | 50 | 60 | 10 |  | ~100 b | (Teng et al., 2020) |

a: as chemical oxygen demand

b: as contaminant removal

S.5.2. Indirect oxidation

The indirect oxidation processes involve the in-situ electro-generation of highly reactive species on the electrode surface (Brillas and Sirés, 2015). These reactive species are produced by the addition of different chemicals like H2O2, Fenton’s reagent, Cl2, HClO, S2O82− and O3 into the aqueous medium (Ridruejo et al., 2018), which was not required in direct oxidation process. Among them, Fenton’s reagent and Cl2 are the most known chemicals used for the production of reactive oxidants species by indirect involvement in the reaction process (Brillas et al., 2005).

S.5.2.1 Electrochemical generation of reactive species by Fenton’s reagent/ Electro-Fenton process

Electro-Fenton based EAOPs was first conceived by Brillas and Oturan’s groups in the year 2000, in which they discovered that •OH can be produced electrochemically with the help of in-situ electro-generation of H2O2 and electro-regeneration of Fe2+ (Eq. 15) (Brillas et al., 2000; Oturan, 2000). Although, the simultaneous generation of H2O2 and regeneration of Fe2+ at cathode surface accelerates the production of •OH in the reaction (Eq. 16), which vigorously reacts with the organic pollutants and completely mineralize the organics into CO2, H2O and inorganic end products. In this method, the production H2O2 is a crucial parameter that influences the efficacy of the treatment system. Therefore, it is required to ensure continuous production of H2O2 via two-electron cathodic reduction of oxygen gas to sustainably treat wastewater in an electrochemical reactor (Eq. 17). The oxygen gas can be directly injected into the electrochemical reactor as pure gaseous form or by purging bubbled air form into the reactor (Sudoh et al., 1986).

Fe2+ + H2O2 → Fe3+ + •OH + OH−						(Eq. 15)

Fe3+ + e− → Fe2+								(Eq. 16)

O2 + 2H+ + 2e− → H2O2							(Eq. 17)

The efficiency and flexibility of indirect EAOPs were successfully reported in investigations on the decontamination of pollutants from the wastewater. For example, a low cost multi-walled carbon nanotubes and carbon black co-modified graphite felt cathode was fabricated for 100 mg L‒1 amoxicillin removal from wastewater and the as-fabricated cathode showed higher H2O2 generation of 309.0 mg L‒1 and simultaneously removed almost 98.7% of amoxicillin by adopting 0.5 mM of Fe2+ catalyst and current density of 15 mA cm‒2 within 30 min of electrolysis (Pan et al., 2020). Similarly, a sacrificial iron anode was used for the removal of malachite green having an initial concentration of 1000 mg L‒1 from wastewater and almost 95.3% of TOC removal was observed by using 0.3 g L‒1 of Fe2+ and current density of 10 mA cm‒2 within 30 min of electrolysis (Teymori et al., 2020). Other 115 articles also addressed the decontamination of organic pollutants by indirect EAOPs according to Scopus database.

In electro-Fenton process, the •OH can be produced electrochemically with the help of in situ electro-generated of H2O2 and electro-regeneration of Fe2+ ions. However, production of these reagent’s depends upon many parameters, such as electrochemical cell configuration, solution conditions, nature of the electrodes and applied current or potential (Jiang et al., 2018). Depending on Fenton’s reagent addition or formation, electro-Fenton process is classified into four types, such as: (1) cathode electro-Fenton process (EF-H2O2); (2) Fe2+ cycling electro-Fenton process (EF-Fere); (3) cathode and Fe2+ cycling electro-Fenton process (EF-H2O2-Fere); and (4) sacrificial anode electro-Fenton process (EF-Feox) (Babuponnusami and Muthukumar, 2014). In EF-H2O2 process, Fe2+ ions are externally added to the system and in situ H2O2 is generated by the electrochemical reduction of O2 on the cathode surface. Therefore, it can reduce the costs and the risk associated with the transport, handling and storage of H2O2 (Badellino et al., 2006). In EF-Feox process, the H2O2 is externally added to the system while Fe2+ ions are electro-generated from sacrificial anode (Eq. 18); whereas in EF-Fere process, both H2O2 and Fe2+ ions are externally added in the electrolytic cell and Fe2+ ions are regenerated through the reduction of Fe3+ ions on the cathode (Eq. 16) (Kurt et al., 2007). Similarly, EF-H2O2-Fere process is the combination of EF-H2O2 and electro-Fenton process, where in situ H2O2 is generated by cathodic reduction of O2 and Fe2+ ions are regenerated through the reduction of Fe3+ ions on the cathode surface (Rahim Pouran et al., 2015) .

Fe → Fe2+ + 2e−								(Eq. 18)

In electro Fenton process, cathode material also plays a significant role to degrade the refractory organic pollutants present in water and wastewater. The efficiency of degradation of refractory organic pollutants majorly depends upon the in-situ H2O2 production on cathode. Scientists have reported that the production of H2O2 can be enhanced by using gas diffusion electrode because of their porous structure which can quickly transfer oxygen and air to the electrode surface (Liu et al., 2019). A low-cost dual gas diffusion cathode was fabricated for efficient H2O2 generation and to degrade 1000 mg L‒1 tartrazine from wastewater. The results explained that H2O2 generation of 566 mg L‒1 and almost 66.1 % of TOC removal can be achieved by adopting 0.4 mM of Fe2+ catalyst at the current density of 7.1 mA cm‒2 after 120 min of treatment (Yu et al., 2015). During another experimentation, activated carbon fiber felt cathode was fabricated for the degradation of antibiotic levofloxacin (200 mg L‒1) from wastewater. The as-fabricated cathode showed COD and TOC removal rates of 79.6% and 61% by employing 1.0 mM of Fe2+ catalyst at a current density of 6.67 mA cm‒2 after 120 min of wastewater treatment (Gong et al., 2016). However, several other materials such as carbon nanotube, graphite, graphite felt etc. have been widely used as cathode in electro-Fenton process for treatment of organic pollutants.

Electro-Fenton process have significant advantages over the conventional Fenton process, such as: (1) rapid regeneration of Fe2+ ions on the cathode reduces the iron sludge production and enhance the degradation efficiency, and (2) on-site generation of H2O2 minimizes the cost and risk associated with the transport, handling and storage of H2O2 (Sirés et al., 2014). However, most of the investigations on electro-Fenton process are conducted in lab-scale and its industrial promotion is limited due to many disadvantages such as low conductivity, low H2O2 generation and concentration polarization in the electrodes (Yuan et al., 2011).

# Supplementary S5

# Table S7. Degradation of contaminants of emerging concern by various Fenton process

| Process | Contaminant | Co (mg L‒1) | Type of Catalyst | Experimental Conditions | Efficiency (%) | Reference |
| --- | --- | --- | --- | --- | --- | --- |
| Heterogeneous Fenton process | p-Arsanilic acid | 10 | SiO2- nZVI | Catalyst dosage = 300 mg L‒1, Time = 60 min | 99.6 | (Lv et al., 2020) |
|  | p-Nitrophenol | 20.9 | Ferrihydrite | Catalyst dosage = 0.2 g L‒1, Time = 5 h | 94.5a | (Wu et al., 2013) |
|  | Methylene blue | 10 | Nb-doped hematite | Catalyst dosage = 10 mg L‒1, Time = 120 min | 75 | (Oliveira et al., 2015) |
|  | Phenol | 47 | Natural Fe clay | Catalyst dosage =1 g L‒1, Time = 6 h | 70a | (Djeffal et al., 2014) |
| Photo-Fenton process | Flutamide | 5 | FeSO4.7H2O | Catalyst dosage = 5 mg L‒1, Time = 120 min | 58 | (Della-Flora et al., 2020) |
|  | 2-Chlorophenol | 1500 | FeCl3·6H2O | Catalyst dosage = 28 mg L‒1, Time = 150 min | 95a | (Poulopoulos et al., 2008) |
|  | Phenol | 50 | Fe-zeolites | Catalyst dosage = 2 g L‒1, Time = 90 min | 90a | (Gonzalez-Olmos et al., 2012) |
| Electro-Fenton process | Florfenicol | 1 | FeCl3 | Catalyst Dosage = n.a.c, Time = 5 h | 90a | (Jiang et al., 2018) |
|  | Phenol | 100 | FeSO4.7H2O | Catalyst dosage = 0.1 mM, Time = 60 min | 93 | (Yang et al., 2012) |
|  | Phenol | 94.11 | FeSO4.7H2O | Catalyst dosage = 0.1 mM, Time = 7 h | ~100a | (Pimentel et al., 2008) |
| Sono-Fenton process | 2-Chlorophenol | 100 | Iron powder | Catalyst dosage= 1 g L‒1, US frequency = 200 kHz, Time = 60 min | ~100 | (Liang et al., 2007) |
|  | Malachite green | 200 | FeSO4.7H2O | Catalyst dosage = 0.5 mM, US frequency = 25 kHz, Time = 60 min | 79.19b | (Melo et al., 2020) |
|  | 2, 4, 6 -Trinitrotoluene | 10 | FeSO4.7H2O | Catalyst dosage = 2 mM, US frequency = 25 kHz, Time = 20 min | ~100 | (Hashemi and Sagharlo, 2020) |
| Photo-electro- Fenton process | 2,4-Dichlorophenoxyacetic acid | 100 | FeSO4 | Catalyst dosage = 1mM, Time = 150 min | 69a | (Badellino et al., 2006) |
|  | 2,4,5-Trichlorophenoxyacetic acid | 226 a | FeSO4.7H2O | Catalyst dosage = 1mM, Time = 6 h | 92a | (Boye et al., 2003) |
|  | Indigo carmine | 900 | FeSO4.7H2O | Catalyst dosage = 1mM, Time=9 h | ~100a | (Flox et al., 2006) |
| Sono-electro- Fenton process | 4,6-Dinitro-o-cresol | 99.06 | FeSO4.7H2O | Catalyst dosage = 0.1 mM, Time = 180 min, US Frequency = 28 kHz | ~100a | (Oturan et al., 2008) |
|  | Red X-GRL dye | 37.5 | FeSO4.7H2O | Catalyst dosage = 5 mM, Time = 180 min, US Frequency = 20 kHz | 76.52a | (Li et al., 2010) |
| Heterogeneous-electro-Fenton process | Vanillic acid | 16.81 | Pyrite | Catalyst dosage= 1 g L‒1, Time = 360 min | 98a | (Ouiriemmi et al., 2017) |
|  | Methyl orange | 50 | Fe(NO3)2· 6H2O | Catalyst dosage = 0.2 mM, Time = 120 min | 93.8a | (Liang et al., 2017) |
|  | Rhodamine B | 10 | Magnetite | Catalyst dosage =10 mg L‒1, Time = 180 min | 97.3a | (Nidheesh et al., 2014) |
| Sono-photo-Fenton process | Reactive Black 5 | 100 | FeSO4.7H2O | Catalyst dosage = 50 mg L‒1, US frequency = 40 kHz, Time = 30 min | 92.2 | (Yosofi and Mousavi, 2020) |
|  | Phenol and Nitrophenol | 260 b 100 a | FeSO4.7H2O | Catalyst dosage = 24 mg L‒1, US frequency = 20 kHz, Time = 60 min | 76.1b COD 68.7a TOC | (Shokri, 2018) |
|  | Azure-B | 40.6 | FeCl3 | Catalyst dosage = 5.0 × 10‒4mol/L, US frequency = 40 kHz, Time= 14 min | ~100 | (Vaishnave et al., 2014) |

a: as total organic carbon

b: as chemical oxygen demand

c: n.a., not available

# Supplementary S8

# Table S8. Degradation of emerging contaminants by non-thermal plasma assisted AOP

| Process | Reactor | Contaminant | Co (mg L‒1) | Time (min) | Efficiency (%) | Reference |
| --- | --- | --- | --- | --- | --- | --- |
| Plasma/TiO2 | Gliding Arc Discharge | Lead acetate | 3253 | 120 | 93 | (Haddou et al., 2014) |
| Plasma/TiO2 | Dielectric barrier discharge | Methyl tert-butyl ether | 88.1 | 10 | ~100 | (Mahyar et al., 2016) |
| Plasma-AOP | Dielectric barrier discharge | Pentoxifylline | 100 | 60 | 92.5 | (Magureanu et al., 2010) |
| Plasma-AOP | Dielectric barrier discharge | Crystal violet | 100 | 10 | 80 | (Reddy and Subrahmanyam, 2012) |
| Plasma/TiO2 | Plasma needle discharge | Reactive Orange 16 | 50 | 60 | ~100 | (Mitrović et al., 2020) |
| Plasma/ N doped TiO2 | Ambient plasma discharge | Methyl blue | 128 | 60 | ~100 | (Ibukun and Jeong, 2020) |
| Plasma-AOP | Atmospheric pressure plasma jet | Telmisartan | 51.5 | 20 | 63 | (Vasua et al., 2019) |

References

Ahmad, A., Priyadarshini, M., Das, S., Ghangrekar, M.M., 2021a. Electrocoagulation as an efficacious technology for the treatment of wastewater containing active pharmaceutical compounds: a review. Sep. Sci. Technol. 1–23. https://doi.org/10.1080/01496395.2021.1972011

Ahmad, A., Priyadarshini, M., Das, S., Ghangrekar, M.M., 2021b. Proclaiming electrochemical oxidation as a potent technology for the treatment of wastewater containing xenobiotic compounds: a mini review. J. Hazardous, Toxic, Radioact. Waste 25, 1–13. https://doi.org/10.1061/(asce)hz.2153-5515.0000616

Arshad, R., Bokhari, T.H., Khosa, K.K., Bhatti, I.A., Munir, M., Iqbal, Mazhar, Iqbal, D.N., Khan, M.I., Iqbal, Munawar, Nazir, A., 2020. Gamma radiation induced degradation of anthraquinone Reactive Blue-19 dye using hydrogen peroxide as oxidizing agent. Radiat. Phys. Chem. 168, 108637. https://doi.org/10.1016/j.radphyschem.2019.108637

Arvaniti, O.S., Frontistis, Z., Nika, M.C., Aalizadeh, R., Thomaidis, N.S., Mantzavinos, D., 2020. Sonochemical degradation of trimethoprim in water matrices: Effect of operating conditions, identification of transformation products and toxicity assessment. Ultrason. Sonochem. 67, 105139. https://doi.org/10.1016/j.ultsonch.2020.105139

Babuponnusami, A., Muthukumar, K., 2014. A review on Fenton and improvements to the Fenton process for wastewater treatment. J. Environ. Chem. Eng. 2, 557–572. https://doi.org/10.1016/j.jece.2013.10.011

Badellino, C., Rodrigues, C.A., Bertazzoli, R., 2006. Oxidation of pesticides by in situ electrogenerated hydrogen peroxide: Study for the degradation of 2,4-dichlorophenoxyacetic acid. J. Hazard. Mater. 137, 856–864. https://doi.org/10.1016/j.jhazmat.2006.03.035

Bagal, M. V., Gogate, P.R., 2013. Degradation of 2,4-dinitrophenol using a combination of hydrodynamic cavitation, chemical and advanced oxidation processes. Ultrason. Sonochem. 20, 1226–1235. https://doi.org/10.1016/j.ultsonch.2013.02.004

Barik, A.J., Gogate, P.R., 2018. Hybrid treatment strategies for 2,4,6-trichlorophenol degradation based on combination of hydrodynamic cavitation and AOPs. Ultrason. Sonochem. 40, 383–394. https://doi.org/10.1016/j.ultsonch.2017.07.029

Basfar, A.A., Muneer, M., Alsager, O.A., 2017. Degradation and detoxification of 2-chlorophenol aqueous solutions using ionizing gamma radiation. Nukleonika 62, 61–68. https://doi.org/10.1515/nuka-2017-0008

Boye, B., Morième Dieng, M., Brillas, E., 2003. Anodic oxidation, electro-Fenton and photoelectro-Fenton treatments of 2,4,5-trichlorophenoxyacetic acid. J. Electroanal. Chem. 557, 135–146. https://doi.org/10.1016/S0022-0728(03)00366-8

Brillas, E., Calpe, J.C., Casado, J., 2000. Mineralization of 2,4-D by advanced electrochemical oxidation processes. Water Res. 34, 2253–2262. https://doi.org/10.1016/S0043-1354(99)00396-6

Brillas, E., Martínez-Huitle, C.A., 2015. Decontamination of wastewaters containing synthetic organic dyes by electrochemical methods. An updated review. Appl. Catal. B Environ. 166–167, 603–643. https://doi.org/10.1016/j.apcatb.2014.11.016

Brillas, E., Sirés, I., 2015. Electrochemical removal of pharmaceuticals from water streams: Reactivity elucidation by mass spectrometry. TrAC - Trends Anal. Chem. 70, 112–121. https://doi.org/10.1016/j.trac.2015.01.013

Brillas, E., Sirés, I., Arias, C., Cabot, P.L., Centellas, F., Rodríguez, R.M., Garrido, J.A., 2005. Mineralization of paracetamol in aqueous medium by anodic oxidation with a boron-doped diamond electrode. Chemosphere 58, 399–406. https://doi.org/10.1016/j.chemosphere.2004.09.028

Cai, C., Duan, X., Xie, X., Kang, S., Liao, C., Dong, J., Liu, Y., Xiang, S., Dionysiou, D.D., 2021. Efficient degradation of clofibric acid by heterogeneous catalytic ozonation using CoFe2O4 catalyst in water. J. Hazard. Mater. 410, 124604. https://doi.org/10.1016/j.jhazmat.2020.124604

Cañzares, P., García-Gómez, J., Sáez, C., Rodrigo, M.A., 2003. Electrochemical oxidation of several chlorophenols on diamond electrodes - Part I. Reaction mechanism. J. Appl. Electrochem. 33, 917–927. https://doi.org/10.1023/A:1025888126686

Čehovin, M., Medic, A., Scheideler, J., Mielcke, J., Ried, A., Kompare, B., Žgajnar Gotvajn, A., 2017. Hydrodynamic cavitation in combination with the ozone, hydrogen peroxide and the UV-based advanced oxidation processes for the removal of natural organic matter from drinking water. Ultrason. Sonochem. 37, 394–404. https://doi.org/10.1016/j.ultsonch.2017.01.036

Changotra, R., Guin, J.P., Varshney, L., Dhir, A., 2018. Assessment of reaction intermediates of gamma radiation-induced degradation of ofloxacin in aqueous solution. Chemosphere 208, 606–613. https://doi.org/10.1016/j.chemosphere.2018.06.003

Chen, H., Zhang, Z., Hu, D., Chen, C., Zhang, Y., He, S., Wang, J., 2021. Catalytic ozonation of norfloxacin using Co3O4/C composite derived from ZIF-67 as catalyst. Chemosphere 265. https://doi.org/10.1016/j.chemosphere.2020.129047

Choi, J., Cui, M., Lee, Y., Kim, J., Son, Y., Lim, J., Ma, J., Khim, J., 2020. Application of persulfate with hydrodynamic cavitation and ferrous in the decomposition of pentachlorophenol. Ultrason. Sonochem. 66, 105106. https://doi.org/10.1016/j.ultsonch.2020.105106

Choi, J., Cui, M., Lee, Y., Ma, J., Kim, J., Son, Y., Khim, J., 2019. Hybrid reactor based on hydrodynamic cavitation, ozonation, and persulfate oxidation for oxalic acid decomposition during rare-earth extraction processes. Ultrason. Sonochem. 52, 326–335. https://doi.org/10.1016/j.ultsonch.2018.12.004

Cuerda-Correa, E.M., Alexandre-Franco, M.F., Fernández-González, C., 2020. Advanced oxidation processes for the removal of antibiotics from water. An overview. Water (Switzerland). https://doi.org/10.3390/w12010102

D, V., A, R., K.N, P., P.V.A, P., V, K., 2019. Degradation of telmisartan (TELMA-H) compounds in aqueous solution using non-thermal atmospheric pressure plasma jet. Front. Adv. Mater. Res. 1, 46–52. https://doi.org/10.34256/famr1919

Della-Flora, A., Wilde, M.L., Thue, P.S., Lima, D., Lima, E.C., Sirtori, C., 2020. Combination of solar photo-Fenton and adsorption process for removal of the anticancer drug Flutamide and its transformation products from hospital wastewater. J. Hazard. Mater. 396, 122699. https://doi.org/10.1016/j.jhazmat.2020.122699

Deng, Y., Zhao, R., 2015. Advanced Oxidation Processes (AOPs) in Wastewater Treatment. Curr. Pollut. Reports 1, 167–176. https://doi.org/10.1007/s40726-015-0015-z

Dewil, R., Mantzavinos, D., Poulios, I., Rodrigo, M.A., 2017. New perspectives for Advanced Oxidation Processes. J. Environ. Manage. 195, 93–99. https://doi.org/10.1016/j.jenvman.2017.04.010

Djeffal, L., Abderrahmane, S., Benzina, M., Fourmentin, M., Siffert, S., Fourmentin, S., 2014. Efficient degradation of phenol using natural clay as heterogeneous Fenton-like catalyst. Environ. Sci. Pollut. Res. 21, 3331–3338. https://doi.org/10.1007/s11356-013-2278-5

Fast, S.A., Gude, V.G., Truax, D.D., Martin, J., Magbanua, B.S., 2017. A Critical Evaluation of Advanced Oxidation Processes for Emerging Contaminants Removal. Environ. Process. 4, 283–302. https://doi.org/10.1007/s40710-017-0207-1

Figueredo, M.A., Rodríguez, E.M., Checa, M., Beltran, F.J., 2019. Ozone-Based Advanced Oxidation Processes for Primidone Removal in Water using Simulated Solar Radiation and TiO2 or WO3 as Photocatalyst. Molecules 24, 1728. https://doi.org/10.3390/molecules24091728

Flox, C., Ammar, S., Arias, C., Brillas, E., Vargas-Zavala, A.V., Abdelhedi, R., 2006. Electro-Fenton and photoelectro-Fenton degradation of indigo carmine in acidic aqueous medium. Appl. Catal. B Environ. 67, 93–104. https://doi.org/10.1016/j.apcatb.2006.04.020

Garcia-Segura, S., Ocon, J.D., Chong, M.N., 2018. Electrochemical oxidation remediation of real wastewater effluents — A review. Process Saf. Environ. Prot. 113, 48–67. https://doi.org/10.1016/j.psep.2017.09.014

Giwa, A., Yusuf, A., Balogun, H.A., Sambudi, N.S., Bilad, M.R., Adeyemi, I., Chakraborty, S., Curcio, S., 2021. Recent advances in advanced oxidation processes for removal of contaminants from water: A comprehensive review. Process Saf. Environ. Prot. 146, 220–256. https://doi.org/10.1016/j.psep.2020.08.015

Gong, Y., Li, J., Zhang, Y., Zhang, M., Tian, X., Wang, A., 2016. Partial degradation of levofloxacin for biodegradability improvement by electro-Fenton process using an activated carbon fiber felt cathode. J. Hazard. Mater. 304, 320–328. https://doi.org/10.1016/j.jhazmat.2015.10.064

Gonzalez-Olmos, R., Martin, M.J., Georgi, A., Kopinke, F.D., Oller, I., Malato, S., 2012. Fe-zeolites as heterogeneous catalysts in solar Fenton-like reactions at neutral pH. Appl. Catal. B Environ. 125, 51–58. https://doi.org/10.1016/j.apcatb.2012.05.022

Guo, Y., Zhao, E., Wang, J., Zhang, X., Huang, H., Yu, G., Wang, Y., 2020. Comparison of emerging contaminant abatement by conventional ozonation, catalytic ozonation, O3/H2O2 and electro-peroxone processes. J. Hazard. Mater. 389, 121829. https://doi.org/10.1016/j.jhazmat.2019.121829

Haddou, N., Ghezzar, M.R., Abdelmalek, F., Ognier, S., Martel, M., Addou, A., 2014. Plasmacatalytic removal of lead acetate assisted by precipitation. Chemosphere. https://doi.org/10.1016/j.chemosphere.2013.12.071

Hashemi, M., Sagharlo, N.G., 2020. Optimization and evaluation of the efficiency of sono-Fenton and photo-Fenton processes in the removal of 2 , 4 , 6 trinitrotoluene ( TNT ) from aqueous solutions. J. Adv. Env. Heal. Res 8, 38–45. https://doi.org/10.22102/jaehr.2020.196050.1139

Hayati, F., Isari, A.A., Anvaripour, B., Fattahi, M., Kakavandi, B., 2020. Ultrasound-assisted photocatalytic degradation of sulfadiazine using MgO@CNT heterojunction composite: Effective factors, pathway and biodegradability studies. Chem. Eng. J. 381, 122636. https://doi.org/10.1016/j.cej.2019.122636

Hoigne, J., Bader, H., 1975. Ozonation of Water: Role of Hydroxyl Radicals as Oxidizing Intermediates. Science (80-. ). 190, 782–784. https://doi.org/10.1126/science.190.4216.782

Hu, J., Bian, X., Xia, Y., Weng, M., Zhou, W., Dai, Q., 2020. Application of response surface methodology in electrochemical degradation of amoxicillin with Cu-PbO2 electrode: Optimization and mechanism. Sep. Purif. Technol. 250, 117109. https://doi.org/10.1016/j.seppur.2020.117109

Ibukun, O., Jeong, H.K., 2020. Enhancement of photocatalytic activities of nitrogen-doped titanium dioxide by ambient plasma. Chem. Phys. Lett. 744, 137234. https://doi.org/10.1016/j.cplett.2020.137234

Iqbal, M., Bhatti, I.A., 2015. Gamma radiation/H2O2 treatment of a nonylphenol ethoxylates: Degradation, cytotoxicity, and mutagenicity evaluation. J. Hazard. Mater. 299, 351–360. https://doi.org/10.1016/j.jhazmat.2015.06.045

Jiang, L., Zhang, L., Chen, J., Ji, H., 2013. Degradation of 17β-estradiol in aqueous solution by ozonation in the presence of manganese(II) and oxalic acid. Environ. Technol. (United Kingdom) 34, 131–138. https://doi.org/10.1080/09593330.2012.689358

Jiang, W.L., Xia, X., Han, J.L., Ding, Y.C., Haider, M.R., Wang, A.J., 2018. Graphene Modified Electro-Fenton Catalytic Membrane for in Situ Degradation of Antibiotic Florfenicol. Environ. Sci. Technol. 52, 9972–9982. https://doi.org/10.1021/acs.est.8b01894

Keykavoos, R., Mankidy, R., Ma, H., Jones, P., Soltan, J., 2013. Mineralization of bisphenol A by catalytic ozonation over alumina. Sep. Purif. Technol. 107, 310–317. https://doi.org/10.1016/j.seppur.2013.01.050

Khan, J.A., Sayed, M., Khan, S., Shah, N.S., Dionysiou, D.D., Boczkaj, G., 2020. Advanced oxidation processes for the treatment of contaminants of emerging concern, in: Contaminants of Emerging Concern in Water and Wastewater. Elsevier, pp. 299–365. https://doi.org/10.1016/B978-0-12-813561-7.00009-2

Khare, P., Patel, R.K., Sharan, S., Shankar, R., 2021. Recent trends in advanced oxidation process for treatment of recalcitrant industrial effluents. Adv. Oxid. Process. Effl. Treat. Plants 137–160. https://doi.org/10.1016/b978-0-12-821011-6.00008-6

Kornienko, G. V., Chaenko, N. V., Maksimov, N.G., Kornienko, V.L., Varnin, V.P., 2011. Electrochemical oxidation of phenol on boron-doped diamond electrode. Russ. J. Electrochem. 47, 225–229. https://doi.org/10.1134/S102319351102011X

Krishnan, R.Y., Manikandan, S., Subbaiya, R., Biruntha, M., Govarthanan, M., Karmegam, N., 2021. Removal of emerging micropollutants originating from pharmaceuticals and personal care products (PPCPs) in water and wastewater by advanced oxidation processes: A review. Environ. Technol. Innov. 23, 101757. https://doi.org/10.1016/j.eti.2021.101757

Kurt, U., Apaydin, O., Gonullu, M.T., 2007. Reduction of COD in wastewater from an organized tannery industrial region by Electro-Fenton process. J. Hazard. Mater. 143, 33–40. https://doi.org/10.1016/j.jhazmat.2006.08.065

Li, G., Yi, L., Wang, J., Song, Y., 2020. Hydrodynamic cavitation degradation of Rhodamine B assisted by Fe3+-doped TiO2: Mechanisms, geometric and operation parameters. Ultrason. Sonochem. 60, 104806. https://doi.org/10.1016/j.ultsonch.2019.104806

Li, H., Lei, H., Yu, Q., Li, Z., Feng, X., Yang, B., 2010. Effect of low frequency ultrasonic irradiation on the sonoelectro-Fenton degradation of cationic red X-GRL. Chem. Eng. J. 160, 417–422. https://doi.org/10.1016/j.cej.2010.03.027

Li, X., Chen, W., Tang, Y., Li, L., 2018. Relationship between the structure of Fe-MCM-48 and its activity in catalytic ozonation for diclofenac mineralization. Chemosphere 206, 615–621. https://doi.org/10.1016/j.chemosphere.2018.05.066

Liang, J., Komarov, S., Hayashi, N., Kasai, E., 2007. Improvement in sonochemical degradation of 4-chlorophenol by combined use of Fenton-like reagents. Ultrason. Sonochem. 14, 201–207. https://doi.org/10.1016/j.ultsonch.2006.05.002

Liang, L., Yu, F., An, Y., Liu, M., Zhou, M., 2017. Preparation of transition metal composite graphite felt cathode for efficient heterogeneous electro-Fenton process. Environ. Sci. Pollut. Res. 24, 1122–1132. https://doi.org/10.1007/s11356-016-7389-3

Lin, N., Gong, Y., Wang, R., Wang, Y., Zhang, X., 2022. Critical review of perovskite-based materials in advanced oxidation system for wastewater treatment: Design, applications and mechanisms. J. Hazard. Mater. 424, 127637. https://doi.org/10.1016/j.jhazmat.2021.127637

Liu, Y.-J., Hu, C.-Y., Lo, S.-L., 2019. Direct and indirect electrochemical oxidation of amine-containing pharmaceuticals using graphite electrodes. J. Hazard. Mater. 366, 592–605. https://doi.org/10.1016/j.jhazmat.2018.12.037

Lv, Y., Huang, S., Huang, G., Liu, Y., Yang, G., Lin, C., Xiao, G., Wang, Y., Liu, M., 2020. Remediation of organic arsenic contaminants with heterogeneous Fenton process mediated by SiO2-coated nano zero-valent iron. Environ. Sci. Pollut. Res. 27, 12017–12029. https://doi.org/10.1007/s11356-020-07808-2

Magureanu, M., Piroi, D., Mandache, N.B., David, V., Medvedovici, A., Parvulescu, V.I., 2010. Degradation of pharmaceutical compound pentoxifylline in water by non-thermal plasma treatment. Water Res. 44, 3445–3453. https://doi.org/10.1016/j.watres.2010.03.020

Mahyar, A., Miessner, H., Mueller, S., Kalass, D., Piechotta, T., Moeller, D., 2016. Non-thermal Plasma Assisted Destruction of Persistent Organic Contaminants in Water - A New Approach Using a Planar Falling Film Reactor. Int. J. Plasma Environ. Sci. Technol. 10, 151–156.

Martínez-Huitle, C.A., Quiroz, M.A., Comninellis, C., Ferro, S., De Battisti, A., 2004. Electrochemical incineration of chloranilic acid using Ti/IrO2, Pb/PbO2 and Si/BDD electrodes. Electrochim. Acta 50, 949–956. https://doi.org/10.1016/j.electacta.2004.07.035

Melo, J.M.O., Duarte, J.L.S., Ferro, A.B., Meili, L., Zanta, C.L.P.S., 2020. Comparing Electrochemical and Fenton-Based Processes for Aquaculture Biocide Degradation. Water, Air, Soil Pollut. 231, 79. https://doi.org/10.1007/s11270-020-4454-9

Meroni, D., Jiménez-Salcedo, M., Falletta, E., Bresolin, B.M., Kait, C.F., Boffito, D.C., Bianchi, C.L., Pirola, C., 2020. Sonophotocatalytic degradation of sodium diclofenac using low power ultrasound and micro sized TiO2. Ultrason. Sonochem. 67, 105123. https://doi.org/10.1016/j.ultsonch.2020.105123

Mitrović, T., Tomić, N., Djukić-Vuković, A., Dohčević-Mitrović, Z., Lazović, S., 2020. Atmospheric Plasma Supported by TiO2 Catalyst for Decolourisation of Reactive Orange 16 Dye in Water. Waste and Biomass Valorization. https://doi.org/10.1007/s12649-019-00928-y

Montoya-Rodríguez, D.M., Serna-Galvis, E.A., Ferraro, F., Torres-Palma, R.A., 2020. Degradation of the emerging concern pollutant ampicillin in aqueous media by sonochemical advanced oxidation processes - Parameters effect, removal of antimicrobial activity and pollutant treatment in hydrolyzed urine. J. Environ. Manage. 261, 110224. https://doi.org/10.1016/j.jenvman.2020.110224

Mora-Gomez, J., Ortega, E., Mestre, S., Pérez-Herranz, V., García-Gabaldón, M., 2019. Electrochemical degradation of norfloxacin using BDD and new Sb-doped SnO2 ceramic anodes in an electrochemical reactor in the presence and absence of a cation-exchange membrane. Sep. Purif. Technol. 208, 68–75. https://doi.org/10.1016/j.seppur.2018.05.017

Nidheesh, P. V., Gandhimathi, R., Velmathi, S., Sanjini, N.S., 2014. Magnetite as a heterogeneous electro Fenton catalyst for the removal of Rhodamine B from aqueous solution. RSC Adv. 4, 5698–5708. https://doi.org/10.1039/c3ra46969g

Oliveira, H.S., Almeida, L.D., De Freitas, V.A.A., Moura, F.C.C., Souza, P.P., Oliveira, L.C.A., 2015. Nb-doped hematite: Highly active catalyst for the oxidation of organic dyes in water. Catal. Today 240, 176–181. https://doi.org/10.1016/j.cattod.2014.07.016

Oturan, M.A., 2000. Ecologically effective water treatment technique using electrochemically generated hydroxyl radicals for in situ destruction of organic pollutants: Application to herbicide 2,4-D. J. Appl. Electrochem. 30, 475–482. https://doi.org/10.1023/A:1003994428571

Oturan, M.A., Aaron, J.J., 2014. Advanced oxidation processes in water/wastewater treatment: Principles and applications. A review. Crit. Rev. Environ. Sci. Technol. https://doi.org/10.1080/10643389.2013.829765

Oturan, M.A., Brillas, E., 2007. Electrochemical Advanced Oxidation Processes (EAOPs) for Environmental Applications. Port. Electrochim. Acta 25, 1–18. https://doi.org/10.4152/pea.200701001

Oturan, M.A., Sirés, I., Oturan, N., Pérocheau, S., Laborde, J.-L., Trévin, S., 2008. Sonoelectro-Fenton process: A novel hybrid technique for the destruction of organic pollutants in water. J. Electroanal. Chem. 624, 329–332. https://doi.org/10.1016/j.jelechem.2008.08.005

Ouiriemmi, I., Karrab, A., Oturan, N., Pazos, M., Rozales, E., Gadri, A., Sanromán, M.Á., Ammar, S., Oturan, M.A., 2017. Heterogeneous electro-Fenton using natural pyrite as solid catalyst for oxidative degradation of vanillic acid. J. Electroanal. Chem. 797, 69–77. https://doi.org/10.1016/j.jelechem.2017.05.028

Pan, G., Sun, X., Sun, Z., 2020. Fabrication of multi-walled carbon nanotubes and carbon black co-modified graphite felt cathode for amoxicillin removal by electrochemical advanced oxidation processes under mild pH condition. Environ. Sci. Pollut. Res. 27, 8231–8247. https://doi.org/10.1007/s11356-019-07358-2

Panda, D., Manickam, S., 2019. Hydrodynamic cavitation assisted degradation of persistent endocrine-disrupting organochlorine pesticide Dicofol: Optimization of operating parameters and investigations on the mechanism of intensification. Ultrason. Sonochem. 51, 526–532. https://doi.org/10.1016/j.ultsonch.2018.04.003

Panda, D., Sethu, V., Manickam, S., 2020. Removal of hexabromocyclododecane using ultrasound-based advanced oxidation process: Kinetics, pathways and influencing factors. Environ. Technol. Innov. 17, 100605. https://doi.org/10.1016/j.eti.2020.100605

Pimentel, M., Oturan, N., Dezotti, M., Oturan, M.A., 2008. Phenol degradation by advanced electrochemical oxidation process electro-Fenton using a carbon felt cathode. Appl. Catal. B Environ. 83, 140–149. https://doi.org/10.1016/j.apcatb.2008.02.011

Poulopoulos, S.G., Nikolaki, M., Karampetsos, D., Philippopoulos, C.J., 2008. Photochemical treatment of 2-chlorophenol aqueous solutions using ultraviolet radiation, hydrogen peroxide and photo-Fenton reaction. J. Hazard. Mater. 153, 582–587. https://doi.org/10.1016/j.jhazmat.2007.09.002

Poyatos, J.M., Muñio, M.M., Almecija, M.C., Torres, J.C., Hontoria, E., Osorio, F., 2010. Advanced oxidation processes for wastewater treatment: State of the art. Water. Air. Soil Pollut. 205, 187–204. https://doi.org/10.1007/s11270-009-0065-1

Rahim Pouran, S., Abdul Aziz, A.R., Wan Daud, W.M.A., 2015. Review on the main advances in photo-Fenton oxidation system for recalcitrant wastewaters. J. Ind. Eng. Chem. 21, 53–69. https://doi.org/10.1016/j.jiec.2014.05.005

Rayaroth, M.P., Aravindakumar, C.T., Shah, N.S., Boczkaj, G., 2022. Advanced oxidation processes (AOPs) based wastewater treatment - unexpected nitration side reactions - a serious environmental issue: A review. Chem. Eng. J. 430, 133002. https://doi.org/10.1016/j.cej.2021.133002

Reddy, P.M.K., Subrahmanyam, C., 2012. Green approach for wastewater treatment-degradation and mineralization of aqueous organic pollutants by discharge plasma. Ind. Eng. Chem. Res. 51, 11097–11103. https://doi.org/10.1021/ie301122p

Ridruejo, C., Centellas, F., Cabot, P.L., Sirés, I., Brillas, E., 2018. Electrochemical Fenton-based treatment of tetracaine in synthetic and urban wastewater using active and non-active anodes. Water Res. 128, 71–81. https://doi.org/10.1016/j.watres.2017.10.048

Rivas-Ortiz, I.B., Cruz-González, G., Lastre-Acosta, A.M., Manduca-Artiles, M., Rapado-Paneque, M., Chávez-Ardanza, A., Teixeira, A.C.S.C., Jáuregui-Haza, U.J., 2017. Optimization of radiolytic degradation of sulfadiazine by combining Fenton and gamma irradiation processes. J. Radioanal. Nucl. Chem. 314, 2597–2607. https://doi.org/10.1007/s10967-017-5629-8

Salimi, M., Esrafili, A., Gholami, M., Jafari, A.J., Kalantary, R.R., Farzadkia, M., Kermani, M., Sobhi, H.R., 2017. Contaminants of emerging concern: a review of new approach in AOP technologies. Environ. Monit. Assess. 189, 414. https://doi.org/10.1007/s10661-017-6097-x

Salmerón, I., Oller, I., Malato, S., 2020. Electro-oxidation process assisted by solar energy for the treatment of wastewater with high salinity. Sci. Total Environ. 705, 135831. https://doi.org/10.1016/j.scitotenv.2019.135831

Scialdone, O., Galia, A., Randazzo, S., 2011. Oxidation of carboxylic acids in water at IrO2–Ta2O5 and boron doped diamond anodes. Chem. Eng. J. 174, 266–274. https://doi.org/10.1016/j.cej.2011.09.016

Senthil Kumar, P., Siva Kumar, M., Pandit, A.., 2000. Experimental quantification of chemical effects of hydrodynamic cavitation. Chem. Eng. Sci. 55, 1633–1639. https://doi.org/10.1016/S0009-2509(99)00435-2

Shokri, A., 2018. Application of Sono–photo-Fenton process for degradation of phenol derivatives in petrochemical wastewater using full factorial design of experiment. Int. J. Ind. Chem. 9, 295–303. https://doi.org/10.1007/s40090-018-0159-y

Sirés, I., Brillas, E., Oturan, M.A., Rodrigo, M.A., Panizza, M., 2014. Electrochemical advanced oxidation processes: Today and tomorrow. A review. Environ. Sci. Pollut. Res. 21, 8336–8367. https://doi.org/10.1007/s11356-014-2783-1

Sudoh, M., Kodera, T., Sakai, K., Zhang, J.Q., Koide, K., 1986. Oxidative degradation of aqueous phenol effluent with electrogenerated fenton’s reagent. J. Chem. Eng. Japan 19, 513–518. https://doi.org/10.1252/jcej.19.513

Sun, M., Yao, Y., Ding, W., Anandan, S., 2020. N/Ti3+ co-doping biphasic TiO2/Bi2WO6 heterojunctions: Hydrothermal fabrication and sonophotocatalytic degradation of organic pollutants. J. Alloys Compd. 820, 153172. https://doi.org/10.1016/j.jallcom.2019.153172

Suslick, K.S., 1990. Sonochemistry. Science (80-. ). 247, 1439–1445. https://doi.org/10.1126/science.247.4949.1439

Teng, J., Liu, G., Liang, J., You, S., 2020. Electrochemical oxidation of sulfadiazine with titanium suboxide mesh anode. Electrochim. Acta 331, 135441. https://doi.org/10.1016/j.electacta.2019.135441

Teymori, M., Khorsandi, H., Aghapour, A.A., Jafari, S.J., Maleki, R., 2020. Electro-Fenton method for the removal of Malachite Green: effect of operational parameters. Appl. Water Sci. 10, 39. https://doi.org/10.1007/s13201-019-1123-5

Thanekar, P., Garg, S., Gogate, P.R., 2020. Hybrid Treatment Strategies Based on Hydrodynamic Cavitation, Advanced Oxidation Processes, and Aerobic Oxidation for Efficient Removal of Naproxen. Ind. Eng. Chem. Res. 59, 4058–4070. https://doi.org/10.1021/acs.iecr.9b01395

Thomas, S., Rayaroth, M.P., Menacherry, S.P.M., Aravind, U.K., Aravindakumar, C.T., 2020. Sonochemical degradation of benzenesulfonic acid in aqueous medium. Chemosphere 252, 126485. https://doi.org/10.1016/j.chemosphere.2020.126485

Tufail, A., Price, W.E., Mohseni, M., Pramanik, B.K., Hai, F.I., 2021. A critical review of advanced oxidation processes for emerging trace organic contaminant degradation: Mechanisms, factors, degradation products, and effluent toxicity. J. Water Process Eng. 40, 101778. https://doi.org/10.1016/j.jwpe.2020.101778

Vaishnave, P., Kumar, A., Ameta, R., Punjabi, P.B., Ameta, S.C., 2014. Photo oxidative degradation of azure-B by sono-photo-Fenton and photo-Fenton reagents. Arab. J. Chem. 7, 981–985. https://doi.org/10.1016/j.arabjc.2010.12.019

Wang, J., Chen, H., Yuan, R., Wang, F., Ma, F., Zhou, B., 2020. Intensified degradation of textile wastewater using a novel treatment of hydrodynamic cavitation with the combination of ozone. J. Environ. Chem. Eng. 8, 103959. https://doi.org/10.1016/j.jece.2020.103959

WANG, J.L., XU, L.J., 2012. Advanced oxidation processes for wastewater treatment: formation of hydroxyl radical and application. Crit. Rev. Environ. Sci. Technol. 42, 251–325. https://doi.org/10.1080/10643389.2010.507698

Wilde, M.L., Montipó, S., Martins, A.F., 2014. Degradation of β-blockers in hospital wastewater by means of ozonation and Fe2+/ozonation. Water Res. 48, 280–295. https://doi.org/10.1016/j.watres.2013.09.039

Wu, C.D., Zhang, Z.L., Wu, Y., Wang, L., Chen, L.J., 2015. Effects of operating parameters and additives on degradation of phenol in water by the combination of H2O2 and hydrodynamic cavitation. Desalin. Water Treat. 53, 462–468. https://doi.org/10.1080/19443994.2013.846234

Wu, Y., Chen, R., Liu, H., Wei, Y., Wu, D., 2013. Feasibility and mechanism of p-nitrophenol decomposition in aqueous dispersions of ferrihydrite and H2O2 under irradiation. React. Kinet. Mech. Catal. 110, 87–99. https://doi.org/10.1007/s11144-013-0571-4

Xia, Y., Wang, G., Guo, L., Dai, Q., Ma, X., 2020a. Electrochemical oxidation of Acid Orange 7 azo dye using a PbO2 electrode: Parameter optimization, reaction mechanism and toxicity evaluation. Chemosphere 241, 125010. https://doi.org/10.1016/j.chemosphere.2019.125010

Xia, Yijing, Bian, X., Xia, Yi, Zhou, W., Wang, L., Fan, S., Xiong, P., Zhan, T., Dai, Q., Chen, J., 2020b. Effect of indium doping on the PbO2 electrode for the enhanced electrochemical oxidation of aspirin: An electrode comparative study. Sep. Purif. Technol. 237, 116321. https://doi.org/10.1016/j.seppur.2019.116321

Xu, B., Qi, F., Sun, D., Chen, Z., Robert, D., 2016. Cerium doped red mud catalytic ozonation for bezafibrate degradation in wastewater: Efficiency, intermediates, and toxicity. Chemosphere 146, 22–31. https://doi.org/10.1016/j.chemosphere.2015.12.016

Xu, L., Tang, S., Wang, K., Ma, X., Niu, J., 2020. Insights into the degradation and detoxication mechanisms of aqueous capecitabine in electrochemical oxidation process. Chemosphere 241, 125058. https://doi.org/10.1016/j.chemosphere.2019.125058

Yang, C., Liu, H., Luo, S., Chen, X., He, H., 2012. Performance of modified electro-Fenton process for phenol degradation using bipolar graphite electrodes and activated carbon. J. Environ. Eng. (United States) 138, 613–619. https://doi.org/10.1061/(ASCE)EE.1943-7870.0000517

Yao, Y., Ren, B., Yang, Y., Huang, C., Li, M., 2019. Preparation and electrochemical treatment application of Ce-PbO2/ZrO2 composite electrode in the degradation of acridine orange by electrochemical advanced oxidation process. J. Hazard. Mater. 361, 141–151. https://doi.org/10.1016/j.jhazmat.2018.08.081

Yosofi, Y., Mousavi, S.A., 2020. Sono-photo-fenton degradation of reactive black 5 from aqueous solutions: Performance and kinetics. Desalin. Water Treat. 174, 354–360. https://doi.org/10.5004/dwt.2020.24843

Yousef Tizhoosh, N., Khataee, A., Hassandoost, R., Darvishi Cheshmeh Soltani, R., Doustkhah, E., 2020. Ultrasound-engineered synthesis of WS2@CeO2 heterostructure for sonocatalytic degradation of tylosin. Ultrason. Sonochem. 67, 105114. https://doi.org/10.1016/j.ultsonch.2020.105114

Yu, S., Hu, J., Wang, J., 2010. Gamma radiation-induced degradation of p-nitrophenol (PNP) in the presence of hydrogen peroxide (H2O2) in aqueous solution. J. Hazard. Mater. 177, 1061–1067. https://doi.org/10.1016/j.jhazmat.2010.01.028

Yu, X., Zhou, M., Ren, G., Ma, L., 2015. A novel dual gas diffusion electrodes system for efficient hydrogen peroxide generation used in electro-Fenton. Chem. Eng. J. 263, 92–100. https://doi.org/10.1016/j.cej.2014.11.053

Yuan, S., Fan, Y., Zhang, Y., Tong, M., Liao, P., 2011. Pd-catalytic in situ generation of H2O2 from H2 and O2 produced by water electrolysis for the efficient electro-fenton degradation of Rhodamine B. Environ. Sci. Technol. 45, 8514–8520. https://doi.org/10.1021/es2022939

Zaouak, A., Noomen, A., Jelassi, H., 2020. Gamma radiation induced degradation of the phenoxy acid herbicide diclofop-methyl in aqueous solutions. Appl. Radiat. Isot. 156, 108939. https://doi.org/10.1016/j.apradiso.2019.108939

Zaouak, A., Noomen, A., Jelassi, H., 2018. Gamma-radiation induced decolorization and degradation on aqueous solutions of Indigo Carmine dye. J. Radioanal. Nucl. Chem. 317, 37–44. https://doi.org/10.1007/s10967-018-5835-z

Zhang, T., Yang, Y., Gao, J., Li, X., Yu, H., Wang, N., Du, P., Yu, R., Li, H., Fan, X., Zhou, Z., 2020. Synergistic degradation of chloramphenicol by ultrasound-enhanced nanoscale zero-valent iron/persulfate treatment. Sep. Purif. Technol. 240, 116575. https://doi.org/10.1016/j.seppur.2020.116575

Zhu, H., Ma, Wencheng, Han, H., Han, Y., Ma, Weiwei, 2017. Catalytic ozonation of quinoline using nano-MgO: Efficacy, pathways, mechanisms and its application to real biologically pretreated coal gasification wastewater. Chem. Eng. J. 327, 91–99. https://doi.org/10.1016/j.cej.2017.06.025

Zhu, X., Tong, M., Shi, S., Zhao, H., Ni, J., 2008. Essential Explanation of the Strong Mineralization Performance of Boron-Doped Diamond Electrodes. Environ. Sci. Technol. 42, 4914–4920. https://doi.org/10.1021/es800298p

Zou, J.-P., Chen, Y., Liu, S.-S., Xing, Q.-J., Dong, W.-H., Luo, X.-B., Dai, W.-L., Xiao, X., Luo, J.-M., Crittenden, J., 2019. Electrochemical oxidation and advanced oxidation processes using a 3D hexagonal Co3O4 array anode for 4-nitrophenol decomposition coupled with simultaneous CO2 conversion to liquid fuels via a flower-like CuO cathode. Water Res. 150, 330–339. https://doi.org/10.1016/j.watres.2018.11.077