Supporting Information

Using multilinear regressions developed from excitation-emission matrices to estimate the wastewater content in urban streams impacted by sanitary sewer leaks and overflows

Jahir A. Batista-Andrade1, Diego Iglesias Vega1, Anna McClain1, and Lee Blaney1*

1:	University of Maryland Baltimore County

Department of Chemical, Biochemical, and Environmental Engineering

1000 Hilltop Circle, Engineering 314

Baltimore, MD 21250 USA

* Corresponding author:

Lee Blaney, PhD

University of Maryland Baltimore County

Department of Chemical, Biochemical, and Environmental Engineering

1000 Hilltop Circle, Engineering Building, Room 314

Baltimore, MD 21250 USA

Tel: +1-410-455-3400

Email: blaney@umbc.edu

Summary:

Number of pages: 24

Number of text sections: 1

Number of tables: 5

Number of figures: 8

Table of contents

Text

Text S1.	CEC analytical methods	S3

Tables

Table S1.	LC-MS/MS parameters for the targeted CECs	S4

Table S2.	Single-parameter linear regressions for the wastewater content of Mixture A and B standards as a function of the area-normalized EEM volumes	S8

Table S3.	Interpretation of the multilinear regression terms in the Mixture A and B calibration standards with 0 – 50% of added wastewater	S9

Table S4.	Estimated wastewater content in samples collected from the Gwynns Falls watershed between April 2019 and March 2020	S10

Table S5.	Estimated wastewater content in samples collected from Gwynns Run (GWN-60) between September 2020 and August 2021	S14

Figures

Figure S1.	Sampling sites in Gwynns Run	S15

Figure S2.	EEM spectra for representative (a) 10 mgC L-1 SRNOM controls (b) field blanks, and (c) laboratory blanks from the April 2023 sample collection	S16

Figure S3.	Mobile phase gradients for analysis of (a) sucralose, carbamazepine, diclofenac, caffeine and paraxanthine, (b) antibiotics, and (c) UV filters, hormones, and ibuprofen	S17

Figure S4.	EEMs measured for Mixture A and B calibration standards generated from samples collected in October 2022, February 2023, and April 2023	S18

Figure S5.	The Φarea–norm,i of each region plotted against the percent of added wastewater in the (left) Mixture A and (right) Mixture B calibration standards generated with samples from (a) October 2022, (b) February 2023, and (c) April 2023	S21

Figure S6.	The matrix-matched calibrations for standards generated from surface water and wastewater collected in (a) October and (b) February using linear regressions with Φ1, Φ2, and Φ4, as appropriate	S22

Figure S7.	Location of sampling sites in the larger Gwynns Falls watershed (Maryland, USA)	S23

Figure S8.	Overlay of wastewater content and sucralose concentrations in surface water samples collected from Gwynns Run between August 2020 and September 2021	S24

Text S1. CEC analytical methods

Oasis hydrophilic-lipophilic balanced cartridges (150 mg, 6 cm3) were acquired from Waters Corp. (Milford, MA, USA) for solid-phase extraction (SPE) of water samples. Liquid chromatography-mass spectrometry (LC-MS) grade water, methanol, and acetonitrile were procured from Fisher Scientific (Hampton, NH, USA). CECs were measured with three methods: (1) sucralose, carbamazepine, diclofenac, caffeine, and paraxanthine; (2) 16 fluoroquinolone, 5 macrolide, 13 sulfonamide, and 7 tetracycline antibiotics; and (3) 13 UV filters, 8 hormones, and ibuprofen. The first two methods employed positive electrospray ionization (ESI), while the third used negative mode. The analytes and parameters for each method are reported in Table S1; note, the UV filter dioxybenzone is listed twice because it was analyzed in both positive and negative ESI mode.

The sample injection volume was 20 µL for all methods. Analytes in method (1) were separated along an XBridge C18 column (3.0×150 mm, 2.5 µm) with a C18 guard column (2.1×5 mm, 3.0 µm). A Waters Symmetry C18 column (2.1×150 mm, 3.5 µm) with a C18 guard column (3.0×10 mm, 3.0 µm) was employed for method (2). In method (3), an XBridge C18 column (2.1×150 mm, 2.5 µm) with a C18 guard column (2.1×5 mm, 3.0 µm) was used for analyte separation. The column compartment was always maintained at 40 °C. All three liquid chromatography with tandem mass spectrometry (LC-MS/MS) methods ran for 12 min, but the mobile phases and elution gradients varied (Figure S3). The mobile phase flow rate was 300 µL min-1 for methods (1) and (2) but 200 µL min-1 in method (3). The gas temperature and flow rate were 300 °C and 5 L min-1, respectively. The nozzle and capillary voltage were 500 V and 3500 V, respectively. The sheath gas flow was 11 L min-1, and the sheath gas temperature was 250 °C for methods (1) and (2) but 350 °C in method (3). The Agilent Mass Hunter Optimizer was used to determine the fragmentor voltage, collision energy, and cell accelerator voltage of the precursor and product ions for methods (1) and (2); however, the analytes in method (3) were manually optimized. The most abundant product ion was used for quantitation, and a separate product ion was used for confirmation.

Table S1. LC-MS/MS parameters for the targeted CECs.

| Chemical | Acronym | Class | MW a | Mode | Precursor | MS-1 b | CE-1 c | MS-2 b | CE-2 c | RT d | IS e |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Method (1): wastewater indicators |  |  |  |  |  |  |  |  |  |  |  |
| Caffeine | CAF | Stimulant | 194.2 | + | 195.1 | 137.8 | 20 | 109.8 | 24 | 2.96 | CAF-d9 |
| Caffeine-d9 | CAF-d9 | - | 203.3 | + | 204.2 | 143.9 | 20 | 115.9 | 28 | 2.94 | - |
| Carbamazepine | CBZ | Pharmaceutical | 236.3 | + | 237.1 | 193.8 | 20 | 178.9 | 56 | 5.52 | CBZ-d10 |
| Carbamazepine-d10 | CBZ-d10 | - | 246.3 | + | 247.2 | 203.9 | 24 | 172.5 | 68 | 5.36 | - |
| Diclofenac | DIC | Pharmaceutical | 296.1 | + | 296.0 | 213.8 | 40 | 150.8 | 80 | 9.67 | DIC-d4 |
| Diclofenac-d4 | DIC-d4 | - | 300.2 | + | 300.1 | 218.0 | 40 | 181.7 | 76 | 9.63 | - |
| Paraxanthine | PAR | Stimulant | 180.2 | + | 181.1 | 124.1 | 20 | 42.2 | 40 | 2.81 | PAR-d6 |
| Paraxanthine-d6 | PAR-d6 | - | 186.2 | + | 187.1 | 127.1 | 24 | 45.2 | 40 | 2.81 | - |
| Sucralose | SUC | Artificial sweetener | 397.6 | + | 419.0 | 239.0 | 15 | 221.1 | 15 | 2.99 | SUC-d6 |
| Sucralose-d6 | SUC-d6 | - | 403.7 | + | 425.0 | 243.0 | 30 | 223.0 | 30 | 2.99 | - |
| Method (2): antibiotics |  |  |  |  |  |  |  |  |  |  |  |
| Azithromycin | AZI | Macrolide | 749.0 | + | 749.5 | 591.4 | 32 | 158.0 | 40 | 7.09 | AZI-d3 |
| Azithromycin-d3 | AZI-d3 | - | 752.0 | + | 752.5 | 594.4 | 32 | 158.0 | 40 | 7.08 | - |
| Chlorotetracycline-13C-d3 | CTC-13C-d3 | - | 482.9 | + | 483.0 | 448.0 | 20 | 466.1 | 16 | 7.02 | - |
| Chlortetracycline f | CTC | Tetracycline | 478.9 | + | 479.1 | 444.0 | 20 | 154.0 | 28 | 7.03 | CTC-13C-d3 |
| Ciprofloxacin | CIP | Fluoroquinolone | 331.4 | + | 332.1 | 231.0 | 44 | 288.0 | 16 | 4.39 | CIP-d8 |
| Ciprofloxacin-d8 | CIP-d8 | - | 339.4 | + | 340.2 | 322.1 | 20 | 235.0 | 44 | 4.33 | - |
| Clarithromycin | CLA | Macrolide | 748.0 | + | 748.5 | 158.0 | 28 | 590.3 | 16 | 8.58 | AZI-d3 |
| Demeclocycline | DMC | Tetracycline | 464.3 | + | 465.1 | 448.0 | 16 | 430.0 | 24 | 6.24 | CTC-13C-d3 |
| Difloxacin f | DIF | Fluoroquinolone | 399.4 | + | 400.2 | 356.1 | 20 | 299.1 | 32 | 5.90 | DIF-d3 |
| Difloxacin-d3 | DIF-d3 | - | 402.4 | + | 403.2 | 385.1 | 24 | 359.1 | 20 | 5.85 | - |
| Doxycycline f | DC | Tetracycline | 444.4 | + | 445.2 | 428.1 | 16 | 321.0 | 32 | 7.53 | CTC-13C-d3 |
| Enrofloxacin f | ENR | Fluoroquinolone | 359.4 | + | 360.2 | 316.1 | 20 | 245.0 | 28 | 4.78 | CIP-d8 |
| Erythromycin | ERY | Macrolide | 733.9 | + | 716.3 | 158.1 | 30 | 558.2 | 10 | 7.90 | ERY-d6 |
| Erythromycin-d6 | ERY-d6 | - | 740.0 | + | 722.4 | 163.9 | 30 | 564.3 | 40 | 7.90 | - |
| Fleroxacin | FLE | Fluoroquinolone | 369.3 | + | 370.1 | 326.1 | 20 | 269.0 | 28 | 2.90 | OFL-d3 |
| Gatifloxacin | GAT | Fluoroquinolone | 375.4 | + | 376.2 | 261.0 | 36 | 332.2 | 16 | 6.66 | DIF-d3 |
| Lomefloxacin | LOM | Fluoroquinolone | 351.4 | + | 352.2 | 265.1 | 24 | 308.2 | 16 | 5.25 | CIP-d8 |
| Marbofloxacin f | MAR | Fluoroquinolone | 362.4 | + | 363.2 | 320.2 | 12 | 205.1 | 36 | 2.65 | OFL-d3 |
| Meclocycline | MC | Tetracycline | 476.9 | + | 477.1 | 460.0 | 20 | 234.9 | 40 | 7.88 | CTC-13C-d3 |
| Methacycline | MTC | Tetracycline | 442.4 | + | 443.2 | 426.1 | 16 | 201.0 | 40 | 7.33 | CTC-13C-d3 |
| Moxifloxacin | MOX | Fluoroquinolone | 401.4 | + | 402.2 | 364.1 | 28 | 220.2 | 40 | 6.98 | DIF-d3 |
| Nadifloxacin | NAD | Fluoroquinolone | 360.4 | + | 361.2 | 283.2 | 44 | 257.2 | 45 | 9.43 | DIF-d3 |
| Norfloxacin | NOR | Fluoroquinolone | 319.3 | + | 320.1 | 231.1 | 44 | 276.2 | 16 | 3.91 | OFL-d3 |
| Ofloxacin | OFL | Fluoroquinolone | 361.4 | + | 362.2 | 318.1 | 20 | 261.0 | 28 | 3.35 | OFL-d3 |
| Ofloxaxin-d3 | OFL-d3 | - | 364.4 | + | 365.2 | 321.1 | 20 | 261.0 | 32 | 3.34 | - |
| Orbifloxacin f | ORB | Fluoroquinolone | 395.4 | + | 396.2 | 352.1 | 20 | 295.0 | 24 | 5.81 | DIF-d3 |
| Oxytetracycline f | OTC | Tetracycline | 460.4 | + | 461.2 | 426.1 | 20 | 443.1 | 12 | 4.14 | TC-d6 |
| Pefloxacin | PEF | Fluoroquinolone | 333.4 | + | 334.2 | 290.1 | 20 | 233.0 | 28 | 3.45 | OFL-d3 |
| Roxithromycin | ROX | Macrolide | 836.5 | + | 837.5 | 679.4 | 20 | 158.0 | 36 | 8.66 | ERY-d6 |
| Sarafloxacin f | SAR | Fluoroquinolone | 385.4 | + | 386.1 | 342.2 | 20 | 299.1 | 32 | 6.41 | DIF-d3 |
| Sparfloxacin | SPA | Fluoroquinolone | 392.4 | + | 393.2 | 349.1 | 20 | 292.1 | 28 | 6.83 | DIF-d3 |
| Sulfacetamide | SCM | Sulfonamide | 214.2 | + | 215.0 | 156.0 | 8 | 92.0 | 24 | 2.26 | SMZ-13C6 |
| Sulfadiazine | SDZ | Sulfonamide | 250.3 | + | 251.1 | 156.0 | 16 | 92.0 | 28 | 2.45 | SMZ-13C6 |
| Sulfadimethoxine f | SDM | Sulfonamide | 310.3 | + | 311.1 | 156.0 | 20 | 92.1 | 36 | 6.95 | SQX-13C6 |
| Sulfadoxine | SDX | Sulfonamide | 310.3 | + | 311.1 | 108.0 | 28 | 140.0 | 28 | 6.95 | SQX-13C6 |
| Sulfamerazine f | SMR | Sulfonamide | 264.3 | + | 265.1 | 92.0 | 32 | 156.0 | 16 | 3.22 | SMZ-13C6 |
| Sulfamethazine f | SDD | Sulfonamide | 278.3 | + | 279.1 | 186.0 | 16 | 92.0 | 32 | 4.31 | SMZ-13C6 |
| Sulfamethizole | SMZ | Sulfonamide | 270.3 | + | 271.0 | 155.9 | 12 | 92.0 | 28 | 4.67 | SMZ-13C6 |
| Sulfamethizole-13C6 | SMZ-13C6 | - | 276.3 | + | 277.0 | 162.0 | 12 | 98.1 | 28 | 4.65 | - |
| Sulfamethoxazole | SMX | Sulfonamide | 253.3 | + | 254.1 | 92.0 | 28 | 155.9 | 16 | 6.79 | SMX-13C6 |
| Sulfamethoxazole-13C6 | SMX-13C6 | - | 259.2 | + | 260.0 | 98.1 | 28 | 162.0 | 16 | 6.80 | - |
| Sulfaphenazole | SPZ | Sulfonamide | 314.4 | + | 315.1 | 158.0 | 32 | 92.1 | 40 | 7.50 | SMX-13C6 |
| Sulfapyridine | SPD | Sulfonamide | 249.3 | + | 250.1 | 156.0 | 16 | 92.1 | 32 | 2.82 | SMZ-13C6 |
| Sulfaquinoxaline f | SQX | Sulfonamide | 300.4 | + | 301.1 | 156.0 | 16 | 92.1 | 36 | 7.81 | SQX-13C6 |
| Sulfaquinoxaline-13C6 | SQX-13C6 | - | 306.3 | + | 307.1 | 162.0 | 16 | 98.1 | 32 | 7.80 | - |
| Sulfathiazole f | STZ | Sulfonamide | 255.3 | + | 256.0 | 156.0 | 12 | 92.0 | 28 | 2.60 | SMZ-13C6 |
| Sulfisomidine | SSD | Sulfonamide | 278.3 | + | 279.1 | 124.1 | 24 | 186.0 | 16 | 1.99 | SMZ-13C6 |
| Tetracycline f | TC | Tetracycline | 444.4 | + | 445.2 | 410.1 | 20 | 154.0 | 28 | 3.74 | TC-d6 |
| Tetracycline-d6 | TC-d6 | - | 450.5 | + | 451.2 | 416.1 | 20 | 160.0 | 28 | 3.69 | - |
| Tosufloxacin | TOS | Fluoroquinolone | 404.4 | + | 405.1 | 263.0 | 52 | 314.0 | 44 | 7.07 | DIF-d3 |
| Tylosin f | TYL | Macrolide | 916.1 | + | 916.5 | 174.0 | 40 | 772.4 | 32 | 7.87 | ERY-d6 |
| Method (3): UV filters, hormones, and ibuprofen |  |  |  |  |  |  |  |  |  |  |  |
| 2-ethyl-d5-hexyl-2,3,3,4,4,5,5,6,6,6-d10 4-methoxycinnamate | EHMC-d15 | - | 305.5 | + | 306.3 | 180.0 | 4 | 161.4 | 20 | 8.51 | - |
| 2-ethylhexyl 4-methoxycinnamate | EHMC | UV filter | 290.4 | + | 291.2 | 179.0 | 4 | 161.0 | 20 | 8.51 | EHMC-d15 |
| 3-(4-methylbenzylidene) camphor | 4-MBC | UV filter | 254.2 | + | 255.2 | 104.9 | 36 | 170.8 | 20 | 8.00 | 4-MBC-d4 |
| 3-(4-methylbenzylidine-d4) camphor | 4-MBC-d4 | - | 258.0 | + | 259.2 | 216.1 | 20 | 108.1 | 32 | 7.98 | - |
| 17α-ethinylestradiol | EE2 | Hormone | 296.4 | - | 295.0 | 145.0 | 40 | 183.0 | 40 | 4.18 | EE2-d4 |
| 17α-ethinylestradiol-2,4,16,16-d4 | EE2-d4 | - | 300.4 | - | 299.0 | 147.0 | 40 | 161.0 | 40 | 4.03 | - |
| 17β-estradiol | E2 | Hormone | 272.4 | - | 271.0 | 183.1 | 45 | 145.1 | 45 | 4.15 | E2-d3 |
| 17β-estradiol-16,16,17-d3 | E2-d3 | - | 275.2 | - | 274.1 | 185.0 | 50 | 145.1 | 45 | 4.11 | - |
| Androsterone | AN | Hormone | 290.4 | + | 291.2 | 273.3 | 10 | 199.1 | 20 | 7.31 | P4-d9 |
| Benzoic acid-d5 | BA-d5 | - | 127.2 | - | 126.0 | 82.1 | 10 | - g | - g | 1.74 | - |
| Benzophenone-d10 | BP-d10 | - | 192.2 | + | 193.1 | 109.9 | 16 | 82.0 | 40 | 4.74 | - |
| Cinoxate | CX | UV filter | 250.3 | + | 251.1 | 160.7 | 4 | 132.8 | 28 | 4.71 | BP-d10 |
| Dioxybenzone | BP-8 | UV filter | 244.3 | - | 243.1 | 123.0 | 20 | 93.1 | 40 | 4.24 | BP-d10 |
| Dioxybenzone | BP-8 | UV filter | 244.3 | + | 245.1 | 120.8 | 16 | 250.8 | 20 | 4.22 | BP-d10 |
| Ensulizole | ESZ | UV filter | 274.3 | - | 273.0 | 193.1 | 30 | 117.2 | 70 | 1.74 | BA-d5 |
| Equilin | EN | Hormone | 268.4 | - | 267.0 | 223.0 | 40 | 142.7 | 40 | 4.17 | E2-d3 |
| Estetrol | E4 | Hormone | 304.4 | - | 303.1 | 240.9 | 15 | 273.0 | 15 | 2.35 | E2-d3 |
| Estriol | E3 | Hormone | 288.4 | - | 287.0 | 171.0 | 40 | 145.1 | 40 | 2.45 | E2-d3 |
| Estrone | E1 | Hormone | 270.1 | - | 269.0 | 145.0 | 40 | 182.9 | 50 | 4.37 | E2-d3 |
| Homosalate | HMS | UV filter | 262.3 | - | 261.3 | 137.0 | 20 | 93.0 | 30 | 8.83 | HMS-d4 |
| Homosalate-(benzoic ring-d4) | HMS-d4 | - | 266.4 | - | 265.2 | 141.0 | 20 | 139.1 | 30 | 8.67 | - |
| Ibuprofen | IBU | Pharmaceutical | 206.3 | - | 205.0 | 160.9 | 5 | - | - | 2.06 | IBU-d3 |
| Ibuprofen-d3 | IBU-d3 | - | 209.3 | - | 208.1 | 164.0 | 5 | - | - | 2.06 | - |
| Octisalate | OS | UV filter | 250.3 | - | 249.1 | 136.9 | 20 | 92.9 | 25 | 8.72 | HMS-d4 |
| Octocrylene | OC | UV filter | 361.5 | + | 362.2 | 249.6 | 4 | 231.6 | 20 | 8.13 | OC-d15 |
| Octocrylene-(2-ethyl-d5-hexyl-2,3,3,4,4,5,5,6,6,6-d10) | OC-d15 | - | 376.6 | + | 377.3 | 233.3 | 20 | 250.6 | 8 | 8.11 | - |
| Oxybenzone | BP-3 | UV filter | 228.2 | + | 229.1 | 150.8 | 20 | 104.9 | 20 | 6.73 | BP3-d5 |
| Oxybenzone-(phenyl-d5) | BP-3-d5 | - | 233.3 | + | 234.1 | 151.0 | 20 | 110.0 | 20 | 6.63 | - |
| Padimate O | ODPABA | UV filter | 277.4 | + | 278.0 | 166.0 | 20 | 150.9 | 40 | 8.46 | OC-d15 |
| Progesterone | P4 | Hormone | 314.5 | + | 315.2 | 97.0 | 25 | 109.0 | 25 | 7.17 | P4-d9 |
| Progesterone-d9 | P4-d9 | - | 323.5 | + | 324.3 | 113.1 | 20 | 288.2 | 20 | 7.13 | - |
| Sulisobenzone | SSB | UV filter | 308.3 | - | 307.0 | 210.9 | 40 | 227.0 | 30 | 1.78 | BA-d5 |
| Trolamine salicylate | TEAS | UV filter | 287.3 | + | 150.1 | 132.1 | 10 | 88.1 | 20 | 2.12 | BA-d5 |

a:  MW, molecular weight (g/mol)

b:  MS-1 was used for quantitation, and MS-2 was used for confirmation

c:  CE, collision energy (V)

d:  RT, retention time (min)

e:  IS, internal standard used for quantitation

f:  animal-labeled antibiotics

g: only one product ion

Table S2. Single-parameter linear regressions for the wastewater content of Mixture A and B standards as a function of the area-normalized EEM volumes. Note, the wastewater content for Mixture A solutions refers to the wastewater added to the standards and does not account for wastewater inherent to the surface water samples.

|  |  | Mixture A |  | Mixture B |  |
| --- | --- | --- | --- | --- | --- |
| Month | Region | Equation | R2 | Equation | R2 |
| October 2022 | 1 | %WWA=0.0468Φi+0.2702 | 0.9978 | %WWB=0.0539Φi+0.1591 | 0.9964 |
|  | 2 | %WWA=0.0981Φi+1.6599 | 0.9979 | %WWB=0.1206Φi+0.3780 | 0.9971 |
|  | 3 | %WWA=0.0519Φi+1.4111 | 0.9985 | %WWB=0.0634Φi+0.4119 | 0.9974 |
|  | 4 | %WWA=0.0451Φi+0.7953 | 0.9992 | %WWB=0.0509Φi+0.2256 | 0.9975 |
|  | 5 | %WWA=0.0193Φi+0.9653 | 0.9965 | %WWB=0.0259Φi+0.2807 | 0.9953 |
| February 2023 | 1 | %WWA=0.1774Φi-0.8198 | 0.9857 | %WWB=0.1354Φi-0.3893 | 0.9969 |
|  | 2 | %WWA=0.1362Φi+0.2551 | 0.9863 | %WWB=0.0959Φi-0.1376 | 0.9974 |
|  | 3 | %WWA=0.0727Φi+0.6893 | 0.9927 | %WWB=0.0599Φi+0.2534 | 0.9989 |
|  | 4 | %WWA=0.0763Φi+0.0857 | 0.9899 | %WWB=0.0661Φi-0.1099 | 0.9983 |
|  | 5 | %WWA=0.0290Φi+0.6039 | 0.9943 | %WWB=0.0288Φi+0.2639 | 0.9950 |
| April 2023 | 1 | %WWA=0.0717Φi+0.2566 | 0.9989 | %WWB=0.0756Φi-0.0615 | 0.9992 |
|  | 2 | %WWA=0.1002Φi+1.0171 | 0.9992 | %WWB=0.1123Φi-0.0253 | 0.9989 |
|  | 3 | %WWA=0.0612Φi+0.9469 | 0.9987 | %WWB=0.0699Φi+0.0970 | 0.9991 |
|  | 4 | %WWA=0.0506Φi+0.5074 | 0.9992 | %WWB=0.0566Φi-0.0162 | 0.9993 |
|  | 5 | %WWA=0.0262Φi+0.6540 | 0.9979 | %WWB=0.0317Φi+0.1075 | 0.9987 |

Table S3. Interpretation of the multilinear regression terms in the Mixture A and B calibration standards with 0 – 50% of added wastewater.

| Percent wastewater (actual) | Percent wastewater (estimated) | β0 | β1Φ1 | β2Φ2 | β3Φ3 | β4Φ4 | β5Φ5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Eq. 10 (main) |  |  |  |  |  |  |
| 0 a | -0.5 ± 0.9 a | -30.4 | -8.5 | +31.3 | -29.9 | +16.6 | +20.4 |
| 5 a | 5.1 ± 0.9 a | -30.4 | -22.8 | +47.2 | -39.5 | +25.9 | +24.7 |
| 10 a | 10.5 ± 0.9 a | -30.4 | -36.9 | +63.5 | -50.3 | +35.2 | +29.4 |
| 20 a | 19.5 ± 0.9 a | -30.4 | -67.1 | +96.0 | -72.9 | +54.4 | +39.4 |
| 30 a | 30.1 ± 0.9 a | -30.4 | -94.6 | +128.0 | -92.7 | +71.9 | +48.0 |
| 40 a | 39.4 ± 0.9 a | -30.4 | -123.8 | +160.3 | -114.5 | +90.5 | +57.4 |
| 50 a | 51.5 ± 0.9 a | -30.4 | -156.1 | +197.3 | -136.8 | +110.8 | +66.8 |
|  | Eq. 11 (main) |  |  |  |  |  |  |
| 0 | 0.0 ± 0.6 | +2.3 | 0.0 | -0.1 | +4.9 | 0.5 | -7.5 |
| 5 | 5.4 ± 0.6 | +2.3 | -3.8 | -9.1 | +23.3 | +12.1 | -19.3 |
| 10 | 10.5 ± 0.6 | +2.3 | -9.2 | -19.8 | +42.5 | +26.1 | -31.4 |
| 20 | 19.6 ± 0.6 | +2.3 | -18.4 | -38.4 | +76.5 | +50.7 | -53.1 |
| 30 | 30.1 ± 0.6 | +2.3 | -28.4 | -56.6 | +113.3 | +76.2 | -76.7 |
| 40 | 39.5 ± 0.6 | +2.3 | -38.1 | -76.2 | +147.4 | +102.1 | -98.1 |
| 50 | 49.2 ± 0.6 | +2.3 | -48.3 | -95.7 | +184.0 | +128.5 | -121.6 |

a: the percent wastewater (actual) for Mixture A standards only refers to the added wastewater and does not account for any wastewater in the surface water component of the mixture; likewise, the percent wastewater (estimated) does not account for wastewater derived from the surface water contents.

Table S4. Estimated wastewater content in samples collected from the Gwynns Falls watershed between April 2019 and March 2020. Note, all estimates include a ±0.55% standard error (not shown for the sake of brevity).

| Site | Month | Uncorrected %WWj (Eq. 13) | Corrected %WWc (Eq. 14) |
| --- | --- | --- | --- |
| GWN-46 | Apr-19 | 3.5 | 2.3 |
| GWN-46 | May-19 | 4.9 | 3.7 |
| GWN-46 | Jun-19 | 4.4 | 3.2 |
| GWN-46 | Jul-19 | 0.0 | 0.0 |
| GWN-46 | Aug-19 | 1.5 | 0.3 |
| GWN-46 | Sep-19 | 1.1 | 0.0 |
| GWN-46 | Oct-19 | 0.0 | 0.0 |
| GWN-46 | Nov-19 | 1.7 | 0.5 |
| GWN-46 | Dec-19 | 0.0 | 0.0 |
| GWN-46 | Jan-20 | 0.7 | 0.0 |
| GWN-46 | Feb-20 | 0.9 | 0.0 |
| GWN-46 | Mar-20 | 0.0 | 0.0 |
| GWN-48 | Apr-19 | 2.3 | 1.1 |
| GWN-48 | May-19 | 7.8 | 6.6 |
| GWN-48 | Jun-19 | 2.6 | 1.4 |
| GWN-48 | Jul-19 | 1.3 | 0.1 |
| GWN-48 | Aug-19 | 1.8 | 0.6 |
| GWN-48 | Sep-19 | 1.5 | 0.3 |
| GWN-48 | Oct-19 | 0.0 | 0.0 |
| GWN-48 | Nov-19 | 1.9 | 0.7 |
| GWN-48 | Dec-19 | 0.0 | 0.0 |
| GWN-48 | Jan-20 | 0.0 | 0.0 |
| GWN-48 | Feb-20 | 0.5 | 0.0 |
| GWN-48 | Mar-20 | 0.0 | 0.0 |
| GWN-49 | Apr-19 | 0.0 | 0.0 |
| GWN-49 | May-19 | 2.7 | 1.5 |
| GWN-49 | Jun-19 | 0.0 | 0.0 |
| GWN-49 | Jul-19 | 0.0 | 0.0 |
| GWN-49 | Aug-19 | 0.8 | 0.0 |
| GWN-49 | Sep-19 | 0.4 | 0.0 |
| GWN-49 | Oct-19 | 0.0 | 0.0 |
| GWN-49 | Nov-19 | 0.6 | 0.0 |
| GWN-49 | Dec-19 | 0.0 | 0.0 |
| GWN-49 | Jan-20 | 0.1 | 0.0 |
| GWN-49 | Feb-20 | 0.0 | 0.0 |
| GWN-49 | Mar-20 | 0.0 | 0.0 |
| GWN-50 | Apr-19 | 4.4 | 3.2 |
| GWN-50 | May-19 | 1.9 | 0.7 |
| GWN-50 | Jun-19 | 6.4 | 5.2 |
| GWN-50 | Jul-19 | 0.0 | 0.0 |
| GWN-50 | Sep-19 | 1.1 | 0.0 |
| GWN-50 | Oct-19 | 0.0 | 0.0 |
| GWN-50 | Dec-19 | 0.0 | 0.0 |
| GWN-50 | Jan-20 | 1.1 | 0.0 |
| GWN-50 | Feb-20 | 1.0 | 0.0 |
| GWN-50 | Mar-20 | 0.0 | 0.0 |
| GWN-51 | Apr-19 | 1.5 | 0.3 |
| GWN-51 | May-19 | 4.3 | 3.1 |
| GWN-51 | Jun-19 | 5.2 | 4.0 |
| GWN-51 | Jul-19 | 0.7 | 0.0 |
| GWN-51 | Aug-19 | 4.4 | 3.2 |
| GWN-51 | Sep-19 | 1.1 | 0.0 |
| GWN-51 | Oct-19 | 0.0 | 0.0 |
| GWN-51 | Nov-19 | 1.2 | 0.0 |
| GWN-51 | Dec-19 | 0.0 | 0.0 |
| GWN-51 | Jan-20 | 0.8 | 0.0 |
| GWN-51 | Feb-20 | 0.8 | 0.0 |
| GWN-51 | Mar-20 | 0.0 | 0.0 |
| GWN-52 | Apr-19 | 2.2 | 1.0 |
| GWN-52 | May-19 | 1.6 | 0.4 |
| GWN-52 | Jun-19 | 5.1 | 3.9 |
| GWN-52 | Jul-19 | 0.1 | 0.0 |
| GWN-52 | Aug-19 | 2.1 | 0.9 |
| GWN-52 | Sep-19 | 2.9 | 1.7 |
| GWN-52 | Nov-19 | 2.2 | 1.0 |
| GWN-52 | Dec-19 | 0.0 | 0.0 |
| GWN-52 | Jan-20 | 0.9 | 0.0 |
| GWN-52 | Feb-20 | 1.2 | 0.0 |
| GWN-52 | Mar-20 | 0.0 | 0.0 |
| GWN-53 | Apr-19 | 3.8 | 2.6 |
| GWN-53 | May-19 | 7.6 | 6.4 |
| GWN-53 | Jun-19 | 8.2 | 7.0 |
| GWN-53 | Jul-19 | 3.2 | 2.0 |
| GWN-53 | Aug-19 | 2.4 | 1.2 |
| GWN-53 | Sep-19 | 2.5 | 1.3 |
| GWN-53 | Oct-19 | 0.0 | 0.0 |
| GWN-53 | Dec-19 | 0.9 | 0.0 |
| GWN-53 | Jan-20 | 1.1 | 0.0 |
| GWN-53 | Feb-20 | 2.0 | 0.8 |
| GWN-53 | Mar-20 | 0.0 | 0.0 |
| GWN-54 | Apr-19 | 6.9 | 5.7 |
| GWN-54 | May-19 | 3.0 | 1.8 |
| GWN-54 | Jun-19 | 10.5 | 9.3 |
| GWN-54 | Jul-19 | 1.4 | 0.2 |
| GWN-54 | Aug-19 | 3.1 | 1.9 |
| GWN-54 | Sep-19 | 2.2 | 1.0 |
| GWN-54 | Oct-19 | 1.9 | 0.7 |
| GWN-54 | Nov-19 | 2.4 | 1.2 |
| GWN-54 | Dec-19 | 3.7 | 2.5 |
| GWN-54 | Jan-20 | 1.6 | 0.4 |
| GWN-54 | Feb-20 | 5.7 | 4.5 |
| GWN-54 | Mar-20 | 36.2 | 35.0 |
| GWN-55 | Apr-19 | 2.4 | 1.2 |
| GWN-55 | May-19 | 4.0 | 2.8 |
| GWN-55 | Jun-19 | 7.4 | 6.2 |
| GWN-55 | Aug-19 | 1.8 | 0.6 |
| GWN-55 | Sep-19 | 2.6 | 1.4 |
| GWN-55 | Oct-19 | 1.2 | 0.0 |
| GWN-55 | Nov-19 | 1.9 | 0.7 |
| GWN-55 | Dec-19 | 4.2 | 3.0 |
| GWN-55 | Jan-20 | 1.0 | 0.0 |
| GWN-55 | Feb-20 | 6.2 | 5.0 |
| GWN-56 | Apr-19 | 7.0 | 5.8 |
| GWN-56 | May-19 | 3.3 | 2.1 |
| GWN-56 | Jun-19 | 5.7 | 4.5 |
| GWN-56 | Aug-19 | 2.7 | 1.5 |
| GWN-56 | Oct-19 | 2.1 | 0.9 |
| GWN-56 | Nov-19 | 1.4 | 0.2 |
| GWN-56 | Dec-19 | 4.4 | 3.2 |
| GWN-56 | Jan-20 | 0.8 | 0.0 |
| GWN-56 | Feb-20 | 4.7 | 3.5 |
| GWN-57 | Apr-19 | 1.8 | 0.6 |
| GWN-57 | May-19 | 4.9 | 3.7 |
| GWN-57 | Jun-19 | 2.2 | 1.0 |
| GWN-57 | Jul-19 | 0.5 | 0.0 |
| GWN-57 | Aug-19 | 1.5 | 0.3 |
| GWN-57 | Sep-19 | 1.8 | 0.6 |
| GWN-57 | Nov-19 | 1.5 | 0.3 |
| GWN-57 | Dec-19 | 1.9 | 0.7 |
| GWN-57 | Jan-20 | 0.5 | 0.0 |
| GWN-57 | Feb-20 | 4.3 | 3.1 |
| GWN-58 | Apr-19 | 4.0 | 2.8 |
| GWN-58 | Jun-19 | 4.8 | 3.6 |
| GWN-58 | Jul-19 | 1.9 | 0.7 |
| GWN-58 | Aug-19 | 2.1 | 0.9 |
| GWN-58 | Sep-19 | 2.0 | 0.8 |
| GWN-58 | Oct-19 | 2.0 | 0.8 |
| GWN-58 | Dec-19 | 1.0 | 0.0 |
| GWN-58 | Jan-20 | 0.6 | 0.0 |
| GWN-58 | Feb-20 | 2.8 | 1.6 |
| GWN-59 | Apr-19 | 2.7 | 1.5 |
| GWN-59 | May-19 | 5.7 | 4.5 |
| GWN-59 | Jun-19 | 5.1 | 3.9 |
| GWN-59 | Aug-19 | 3.1 | 1.9 |
| GWN-59 | Sep-19 | 2.6 | 1.4 |
| GWN-59 | Oct-19 | 1.8 | 0.6 |
| GWN-59 | Dec-19 | 1.9 | 0.7 |
| GWN-59 | Jan-20 | 0.7 | 0.0 |
| GWN-59 | Feb-20 | 2.8 | 1.6 |
| GWN-60 | Apr-19 | 10.3 | 9.1 |
| GWN-60 | May-19 | 6.6 | 5.4 |
| GWN-60 | Jun-19 | 14.1 | 12.9 |
| GWN-60 | Jul-19 | 5.4 | 4.2 |
| GWN-60 | Aug-19 | 5.2 | 4.0 |
| GWN-60 | Sep-19 | 6.3 | 5.1 |
| GWN-60 | Oct-19 | 4.5 | 3.3 |
| GWN-60 | Nov-19 | 3.7 | 2.5 |
| GWN-60 | Dec-19 | 7.2 | 6.0 |
| GWN-60 | Jan-20 | 0.4 | 0.0 |
| GWN-60 | Feb-20 | 6.0 | 4.8 |

Table S5. Estimated wastewater content in samples collected from Gwynns Run (GWN-60) between September 2020 and August 2021. Note, all estimates include a ±0.55% standard error (not shown for the sake of brevity).

| Site | Month | Uncorrected %WWj (Eq. 13) | Corrected %WWc (Eq. 14) |
| --- | --- | --- | --- |
| GWN-60 | Sep-20 | 11.3 | 10.1 |
| GWN-60 | Oct-20 | 7.2 | 6.0 |
| GWN-60 | Nov-20 | 7.9 | 6.7 |
| GWN-60 | Dec-20 | 6.4 | 5.2 |
| GWN-60 | Jan-21 | 6.2 | 5.1 |
| GWN-60 | Feb-21 | 3.5 | 2.3 |
| GWN-60 | Mar-21 | 8.8 | 7.6 |
| GWN-60 | Apr-21 | 5.7 | 4.5 |
| GWN-60 | May-21 | 8.3 | 7.1 |
| GWN-60 | Jun-21 | 20.2 | 19.0 |
| GWN-60 | Jul-21 | 14.9 | 13.7 |
| GWN-60 | Aug-21 | 10.6 | 9.4 |

Figure S1. Sampling sites in Gwynns Run. Surface water was collected from GWN-60, and wastewater was retrieved from the manhole at WW-1. Discharge data from USGS 01589352 were used to inform the relative stream flow between sampling times. Reference: USGS, Surface-Water Daily Data for the Nation: USGS 01589352 Gwynns Falls at Washington Blvd at Baltimore, MD. Accessed July 2023.

Figure S2. EEM spectra for representative (a) 10 mgC L-1 SRNOM solution (b) field blanks, and (c) laboratory blanks from the April 2023 sample collection. White lines represent the boundaries of Regions 1, 2, 3, 4, and 5 defined in (a) and described in the main manuscript. The SRNOM solution was run after every six samples for quality assurance and control purposes. Field and laboratory blanks did not exhibit any notable fluorescence.

Figure S3. Mobile phase gradients for analysis of (a) sucralose, carbamazepine, diclofenac, caffeine and paraxanthine, (b) antibiotics, and (c) UV filters, hormones, and ibuprofen. Fresh mobile phase was prepared before every analysis.

Figure S4 (below and continued to page S19). EEMs measured for Mixture A and B calibration standards generated from samples collected in October 2022, February 2023, and April 2023. All wastewater (WW) contents are expressed as percent volume. Except for the 100% WW samples, all solutions contained 0.5 mgC L-1 SRNOM. The surface water (Mixture A), deionized water (Mixture B), and municipal wastewater contents were varied as suggested by the listed %WW values.

Mixture A, October 2022 – Surface water, SRNOM, wastewater

Mixture B, October 2022 – Deionized water, SRNOM, wastewater

[see Figure 1 in main manuscript]

Mixture A, February 2023 – Surface water, SRNOM, wastewater

Mixture B, February 2023 – Deionized water, SRNOM, wastewater

Mixture A, April 2023 – Surface water, SRNOM, wastewater

Mixture B, April 2023 – Deionized water, SRNOM, wastewater

Figure S5. The Φarea–norm,i of each region plotted against the percent of added wastewater in the (left) Mixture A and (right) Mixture B calibration standards generated with samples from (a) October 2022, (b) February 2023, and (c) April 2023. All relationships followed linear trends (R2 > 0.985). More details of the linear regressions are available in Table S2.

Figure S6. The matrix-matched calibrations for standards generated from surface water and wastewater collected in (a) October and (b) February using linear regressions with Φ1, Φ2, and Φ4, as appropriate. The plots are for the significant variables from multilinear regressions of Mixture A calibration standards in Table 2 (main manuscript).

Figure S7. Location of sampling sites in the larger Gwynns Falls watershed (Maryland, USA). Samples were collected from Gwynns Run (GWN-60) during three individual campaigns between April 2019 and April 2023. For the other sites, surface water samples were collected between April 2019 and March 2020.

Figure S8. Overlay of wastewater content and sucralose concentrations in surface water samples collected from Gwynns Run between August 2020 and September 2021. The wastewater contents were estimated with the multilinear regression in Eq. 14.