Supporting Information

Comparative Study of CO₂ Nanobubbles and Macrobubbles: Effects on Water Chemistry, Microalgal Growth, and Carbon Utilization

Lili Li1, 2, 3, Jingru Wei1, Yi-Ying Lee4, Yihan Zhang1, Shan Xue1, 5, Sowmya Atukuri1, Yantao Li4, Taha Marhaba1, Xuezhi Zhang2, Wen Zhang1*

1 John A. Reif, Jr. Department of Civil and Environmental Engineering, New Jersey Institute of Technology, Newark, New Jersey 07102, United States

2 Institute of Hydrobiology, Chinese Academy of Sciences, Wuhan 430072, China

3 University of Chinese Academy of Sciences, Beijing 100049, China

4 Institute of Marine and Environmental Technology, University of Maryland Center for Environmental Science, Baltimore, Maryland 21202, United States

5 PureNanotech Inc. 239 New Rd A 104, Parsippany, New Jersey 07054, United States

*Corresponding author: Wen Zhang (wen.zhang@njit.edu)

Table of Contents

| Section S1 | CO2 nanobubble generation setup |
| --- | --- |
| Section S2 | Calculation of CO₂ transfer efficiency |
| Section S3 | Algae cultivation device |
| Section S4 | Time-dependent effects of 10% CO₂ nanobubble generation modes on water chemistry |
| Section S5 | Algal biomass sedimentation under limited aeration |
| Section S6 | Statistical analysis |

Section S1. CO2 Nanobubble generation setup

Figure S1. Circulation mode for CO₂ nanobubble generation: (a) experimental setup; (b) schematic diagram.

Figure S2. Direct injection mode for CO₂ nanobubble generation: (a) ceramic membrane; (b) experimental setup; (c) schematic diagram.

Section S2. Calculation of CO₂ transfer efficiency

According to the ideal gas law, the total input CO2 was calculated using Eq.(S1-S3).

(S1)

(S2)

(S3)

where nCO2 and MCO2 are the mole amount of CO2 (mol) and the molar mass of CO2 (44.01 g·mol-1), respectively. P is the partial pressure (4.08 atm), VCO2 is the gas volume (L), R is the ideal gas constant (0.0821 L·atm·mol-1·K-1), T is the absolute temperature (298 K), Q is the gas flow rate (0.45 L·min-1), and t is the duration of gas introduction (min).

(S4)

(S5)

(S6)

where CCO2-water is the dissolved CO2 concentration in water (g·L-1), Vliquid is the liquid volume (0.5 L), r is the radius of nanobubbles (60 nm), CCO2-NBs is the nanobubble concentration (2×108 particles·mL-1),  is the density of CO2 nanobubbles at 4 atm (7333.90 g·m-3). In this study, the nanobubble diameter was approximately 120 nm, resulting in extremely low individual volume and total gas content due to the high surface-to-volume ratio. Compared to the mass of dissolved CO2 in water, the amount of CO2 inside the nanobubbles is negligible. Therefore, the contribution of mCO2-nanobubble was considered negligible in this calculation.

Section S3. Algae cultivation device

Figure S3. Experimental setup for microalgae cultivation.

Section S4. Time-dependent effects of 10% CO₂ nanobubble generation modes on water chemistry

Figure S4. Time-dependent changes in (a) dissolved CO2 concentration, (b) DO concentration, (c) pH, and (d) redox potential during purging with 10% CO2 gas in DI water using nanobubbles from circulation and direct modes, with macrobubbles as the control.

Section S5. Algal biomass sedimentation under non-aerated conditions

Figure S5. Changes in algal cell concentration (OD750) in the supernatant under non-aerated conditions over time.

Section S6. Statical analysis

All experiments were conducted at room temperature, and the results are presented as mean values ± standard deviations. Error bars represent the standard deviation based on three experimental replicates. Statistical differences between the experimental and control groups were assessed using one-way ANOVA with a significance level of α = 0.05. The data were tested for normality using the Shapiro-Wilk test before performing the one-way ANOVA. Based on these tests, all datasets were found to follow a normal distribution, ensuring the validity of the ANOVA results. All data visualizations and processing were performed using Origin version 2025b.

Table S1. p values from the ANOVA test comparing OD750 over the 14-day cultivation period.

|  |  | CO2 nanobubble |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 3 | 6 | 9 | 11 | 13 | 14 |  |
| CO2 macrobubble | 1 | NA |  |  |  |  |  |  |  |
|  | 3 |  | NA |  |  |  |  |  |  |
|  | 6 |  |  | NA |  |  |  |  |  |
|  | 9 |  |  |  | 0.0075 |  |  |  |  |
|  | 11 |  |  |  |  | 0.0037 |  |  |  |
|  | 13 |  |  |  |  |  | ＜0.0001 |  |  |
|  | 14 |  |  |  |  |  |  | 0.0003 |  |

Table S2. p values from the ANOVA test comparing carbon utilization efficiency under different aeration modes.

|  |  | Nanobubble |  |
| --- | --- | --- | --- |
|  |  | 10% CO2 | 100% CO2 |
| Macrobubble | 10% CO2 | 0.0172 |  |
|  | 100% CO2 |  | NA |

Table S3. p values from the ANOVA test comparing pH variation over the 14-day cultivation period.

|  |  | CO2 nanobubble |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 3 | 6 | 9 | 11 | 13 | 14 |  |
| CO2 macrobubble | 1 | 0.0007 |  |  |  |  |  |  |  |
|  | 3 |  | 0.0040 |  |  |  |  |  |  |
|  | 6 |  |  | 0.0069 |  |  |  |  |  |
|  | 9 |  |  |  | 0.0047 |  |  |  |  |
|  | 11 |  |  |  |  | ＜0.0001 |  |  |  |
|  | 13 |  |  |  |  |  | 0.0102 |  |  |
|  | 14 |  |  |  |  |  |  | ＜0.0001 |  |

Table S4. p values from the ANOVA test comparing dissolved CO2 concentrations over the 14-day cultivation period.

|  |  | CO2 nanobubble |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 3 | 6 | 9 | 11 | 13 | 14 |  |
| CO2 macrobubble | 1 | 0.001 |  |  |  |  |  |  |  |
|  | 3 |  | 0.0047 |  |  |  |  |  |  |
|  | 6 |  |  | 0.0226 |  |  |  |  |  |
|  | 9 |  |  |  | 0.0111 |  |  |  |  |
|  | 11 |  |  |  |  | NA |  |  |  |
|  | 13 |  |  |  |  |  | NA |  |  |
|  | 14 |  |  |  |  |  |  | NA |  |